May 18, 2007

EnCana Oil & Gas (USA) Inc.

370 17th St., Suite 1700

Denver, CO 80202

Attn:  Mr. Chris Williams

Dear Mr. Williams:

Cordilleran Compliance Services, Inc. (Cordilleran) has been retained by EnCana Oil & Gas (USA) Inc. to perform technical environmental services including; quarterly groundwater monitoring, surface-water assessment, and remediation of groundwater that has been impacted by dissolved phase volatile hydrocarbons comprised primarily of methane and benzene in the area of the West Divide Creek Gas Seep (Figure 1).

The objective of the continued operation of the remediation system and quarterly water sampling is to mitigate and control migration of the dissolved phase hydrocarbons in the down gradient direction and into nearby Divide Creek and surrounding area groundwater wells, and to treat the source of the hydrocarbons.

This report summarizes the results of surface-water and groundwater monitoring in March 2007 at the seep site in a continued effort to monitor the possible migration of the benzene plume prior to the COGCC staff rendering a decision on the reduction of the sampling frequency.

Groundwater and Surface-Water Monitoring

Samples collected during this period were analyzed by Evergreen Analytical Laboratory (EAL) of Wheat Ridge, CO for the following analysis:

- BTEX/MTBE using EPA method 8021

- Dissolved methane using method RSK 175M

- Chloride (Cl) using method 300E

- Sodium (Na) using method SW6020

- pH using EPA method 150.1

- Total dissolved solids (TDS) using method SM240C

- Specific conductivity using method SM251B.

Isotopic methane was determined by Isotech Labotories, Inc of Champaign, IL. Stable isotopes of carbon and hydrogen in methane and stable isotopes of carbon in ethane and propane and gas composition were determined where dissolved gasses were sufficient.

Cordilleran collected groundwater samples from twenty-four monitoring wells during March 2007. Twenty of these monitoring wells are located on the Langegger property, one well is located on the Thompson property, and three wells are located on the Eicher property. Groundwater samples and field parameters (temperature, specific conductance, dissolved oxygen, pH, total dissolved solids and turbidity), were collected from March 12, 2007 through March 13, 2007 from monitoring wells (MW-1, 2, 4, 6-9, 11-18, 20-25, 26 and 27) (Figure 1). Prior to sample collection, static water levels were measured in monitoring wells to within 0.01 feet (ft) from the top of the PVC casing using an electronic water level indicator. Groundwater elevations are graphically illustrated in Figure 2. A total of three casing volumes were removed prior to sampling each well using dedicated disposable bailers with bottom loading valve assemblies. Field parameters were obtained at the completion of purging activities (Appendix A). Groundwater samples were collected following field parameter measurements.

Cordilleran collected eight surface-water samples (DCS-1-8) and field parameters (temperature, specific conductance (SpC), dissolved oxygen, pH, total dissolved solids, and turbidity) from the West Divide Creek located on the Langegger property on March 12, 2007. Groundwater and surface-water samples were placed in the appropriate sample containers provided by EAL, labeled, stored on ice, and delivered under chain-of-custody procedures to EAL. Groundwater and surface-water samples were analyzed for benzene, toluene, ethylbenzene, total xylenes, methyl tertiary-butyl ether, dissolved methane, chloride, sodium, total dissolved solids, and SpC.

Site Hydrogeology and Hydrology

In the vicinity of the seep area, groundwater was encountered at depths ranging from surface level to 23.6 feet below ground surface (ft-bgs). The groundwater flow direction continues to be from the seep area towards the north, mimicking the creek flow direction. The shallow, unconfined groundwater is in communication with surface water of West Divide Creek from the east side of the creek and discharges to groundwater on the west side of the creek, and is generally of good quality. Groundwater found in the area, east of West Divide Creek is influenced by springs that originate from unlined irrigation ditches located on the mesa to the east.

The groundwater flow velocity was determined to be 0.015 feet/foot (ft/ft), which is low compared to previous studies during March. The interaction between groundwater and the creek based on water level measurements to evaluate water gain/loss was determined by measuring water levels in the stream and piezometers on the stream banks. The results generally indicate that the creek is losing water to groundwater on the west side of the creek and the creek is gaining water from the wetlands east of the creek.

The flow in the creek during March is generally starting to gain discharge from spring runoff followed by high flow into the middle of spring. During March, the south-east side of the study area primarily within the vicinity of the wetlands near monitoring wells 12, 13, 19 and 25 the area was frozen and surface runoff is mixing with the groundwater within this area; therefore the hydraulic head was higher than average within this area.

Benzene

Monitoring wells 2, 4, 9 and 17 have benzene concentrations of 230μgl, 172μgl, 66μgland 7.5μgl  respectively, which exceed the Colorado Basic Ground Water Quality (CBGWQ) standard of 5μgl. The laboratory results for benzene concentrations for each monitoring well are summarized in Table 1 in Appendix A. Benzene concentrations are graphically illustrated in Figure 3. No other monitoring wells showed benzene concentration above the standard. The size of the area underlain by groundwater that is impacted by benzene at concentrations above 1.0μgl is approximately 89,893 ft2.

Toluene

Toluene was not detected above the lower laboratory detection limit (LDL) 2μgl in any monitoring wells during March of 2007 (Table 1).

Ethylbenzene

Ethylbenzene was present in wells MW-2 and MW-4 at a concentration of 5.8μgl and 6.73μgl respectively. These concentrations are below the CBGWQ standard of 680μgl. Ethylbenzene was not detected in any other monitoring wells (Table 1).

Total Xylenes

Total Xylenes were present in wells MW-2, MW-4 and MW-9 at concentrations of 37.8μgl, 69.28μgl and 2μgl respectively. These concentrations are below the CBGWQ groundwater standard of 10,000μgl. Total xylenes were not detected in any other monitoring wells. The laboratory results for total xylenes concentrations are summarized in Table 1.

MTBE

Laboratory results indicate that MTBE (methyl tertiary-butyl ether) was not present above the LDL 4μgl in the groundwater samples collected in the monitoring area during March 2007 (Table 1).

Methane

Total dissolved methane concentrations in groundwater have decreased since the start up of the air sparge system in the area of MW-1, MW-6, MW-7, and MW-8. Total dissolved methane concentrations above the LDL was detected in monitoring wells MW-1, 2, 4, 6-9, 11-18, 23, 25 and 26. In general the highest concentrations of dissolved methane in the groundwater are located near the seep. Total dissolved methane concentrations are graphically illustrated in Figure 4. In the remaining wells, dissolved methane was not detected above the LDL. Total dissolved methane is summarized in Table 1.

Thermogenic Methane

The laboratory results for methane were reported as total dissolved methane. This included both biogenic (methane gas generated by biologic reduction of organic matter) and thermogenic methane (methane gas generated by thermal reduction of deeply buried organic matter). Using the reported total dissolved methane concentration, the concentration of thermogenic methane was calculated. Hydrocarbon gas from ‘biogenic only’ sources contains a high proportion of methane (>99%) and has characteristic carbon and hydrogen isotopes ratios. Typically, thermogenic methane is indicated by isotope ratios that are less negative than ratios for biogenic methane. To estimate the fraction of total methane in a water sample that can be attributed to thermogenic sources, an algebraic mixing calculation was used after the data were evaluated on the basis of the laboratory-determined values of methane carbon-13 isotope ratioδ13C1, methane hydrogen isotope ratio δDC1 and ratio of methane to ethane and propaneC1C2+C3.

Initially ‘biogenic-only’ sources are easily identified by comparing the laboratory data to literature values of the parameters discussed above. The methane fraction of a ‘reservoir-typical’ thermogenic source (79%) is used as a baseline in the mixing calculations. The biogenic source is assumed to be 100% methane and then an algebraic mixing calculation is used to determine what percentage of the total methane comes for biogenic versus thermogenic sources.

Isotopic samples were collected from monitoring wells 2, 4, 9, 12, 14, 16, 17, 18 and 23 during March 2007 sampling. These wells have shown high concentration of dissolved methane in the past. The remaining wells have showed low concentrations of dissolved methane. A compilation of isotopic data was analyzed from September 2004 to March 2007 for all monitoring stations with significant amount of methane to understand the thermogenic methane in the subsurface. Graphical results (Figures 5-7) indicate that thermogenic methane has fluctuated but remained the same in the area upgradient of the remediation system and near the seep. Thermogenic methane concentrations have dropped dramatically down gradient of the seep and within the remediation system.

Surface-Water Quality

Laboratory results indicate that BTEX/MTBE compounds were not detected in any of the Divide Creek surface-water samples. Dissolved methane was detected in all samples excluding DCS1; all with concentrations measuring less than 0.1 mgl (Table 1). The only area where thermogenic methane is present is within and near the vicinity of the seep. DCS-2 had a thermogenic methane concentration of 0.042mgl. A compilation of isotopic data was analyzed from September 2004 to March 2007 for DCS-2 and DCS-3 (Figure 7). Graphical results indicate that thermogenic methane has dropped considerably within and down stream of the seep at DCS-2 and DCS-3; although during March 2007 a slight increase in thermogenic methane in DCS-2 is graphically illustrated in figure 7.

Inorganic Results

Monitoring well and surface-water results for inorganic constituents, chloride (Cl), sodium (Na), total dissolved solids (TDS), pH, and specific conductivity (SpC) are shown in Appendix A. The Divide Creek study area water continues to be high in TDS and sodium which can be attributed to the high amount of dissolved solids within the Wasatch Formation groundwater. Chloride, specific conductivity, and pH are within normal ranges as previously observed during other monitoring periods. To fully understand the interconnection between the groundwater and West Divide Creek an extended (alkalinity, cations and anions) analysis would be necessary.

QA/QC

Laboratory and field quality assurance and quality control (QA/QC) consisted of analyzing duplicate samples, matrix spikes and duplicate analyses. For quality assurance, duplicate water samples were acquired at an approximate rate of 1 for every 10 samples. In addition replicate/split samples were also obtained at an approximate rate of 1 for every 10 samples. Replicate/split samples were sent to ESN Rocky Mountain, Golden, Colorado. During March 2007 sampling event three duplicate and three replicate/split samples were obtained. Analytical results indicate insignificant differences between actual and replicate samples. Analytical results from ESN indicated that monitoring wells 9 and 14 had a MTBE concentration of 6.1μgl and 4μgl respectively. A re-sample was conducted on April 30, 2007 on monitoring wells 9 and 14, analytical results indicate that the MTBE hit was due to lab error. Laboratory analytical reports and chain-of-custody forms are included in Appendix B.

Divide Creek Seep Status

The air sparge remediation system has operated continuously throughout the quarter with minimal down time. Since the start up of the system in April of 2004, the number of monitoring stations with benzene concentrations above the maximum contaminant level (MCL) in the area has been reduced from 14 to 4. The size of the area underlain by groundwater that is impacted by benzene concentrations above 1.0μgl has been reduced from 134,974 ft2 in June of 2005 to approximately 89,893 ft2 in March of 2007, a total reduction of 45,081 ft2. Benzene was not detected in any well located within or down gradient of the remediation system during the March 2007 sampling event. Benzene greater than 5μgl in the groundwater is primarily located within 300 feet of the seep (Figure 3). Thermogenic methane has fluctuated but remained the same in the vicinity of the seep. Thermogenic methane concentrations have dropped significantly down stream of the seep and in area of the remediation system. BTEX/MTBE was not detected in the surface-water samples. Dissolved methane was not detected above 0.11mgl  in the surface water samples. Dissolved methane in the groundwater greater than 1.0mgl is predominantly found within 200 feet of the seep. Thermogenic methane was present at one station within West Divide Creek (DCS-2) and near the seep at a minor concentration of 0.042mgl.

The size of the plume comprised primarily of methane and benzene has decreased since the start of the air sparge remediation system and dissolved oxygen readings have increased since the implementation. Therefore, the remediation system has been effectively mitigating the plume. A negative thermogenic methane trend is apparent in monitoring wells (2, 4, 9, 12, 17, 18,) and West Divide Creek monitoring station 3.

Cordilleran recommends that the system be extended to include the area in the near vicinity of wells MW-16 and MW-17 which have shown benzene concentrations exceeding the standard. In addition, during the third quarter of 2007, Cordilleran will begin to implement a reduction in remediation treatment times is order to optimize the system. If any benzene is detected during this time at well MW-7, the system will be returned to full treatment time. It is expected that the treatment time will be reduced from 24hr/day to 12hr/day. The addition of an added sparge well nest will further aid in the process of remediating the plume and mitigating the seep. This action would ensure that the benzene plume would be constricted to a narrower area; and the contaminated groundwater area would likely decrease dramatically within a shorter period of time.

Comprehensive monitoring and remediation has been implemented since March 2004. Monitoring of all springs, ponds, monitoring wells, and West Divide Creek has continuously shown that the air sparge remediation system has successfully confined the hydrocarbon plume. Cordilleran has reviewed 36 months of data collected since the West Divide Creek Gas Seep discovery in March of 2004. Monitoring wells (13, 15, 19, 20, 21, 22, 23, 24, 25, and 26) have not shown BTEX/MTBE concentrations above the detection limits since their installation. It is Cordilleran’s recommendation that these wells be tested on an annual basis. Due to the site hydrology and continued operation of the remediation system with the added assurance of additional air sparge wells, benzene migration to monitoring wells (13, 15, 19, and 20-26) or downgradient of the system is unlikely. All remaining monitoring stations at West Divide Creek and monitoring wells should continue to be monitored on a quarterly basis.

Cordilleran appreciates the opportunity to provide services to EnCana Oil & Gas (USA) Inc.  If you have any questions or concerns regarding this information, please contact our offices.

Sincerely,

Cordilleran Compliance Services, Inc.

Prepared by:                                					           Reviewed by

Scotty Mann								           Dion Plsek, P.E

Field Technician							           Principal Engineer

Brad Stephenson, P.G.

Senior Hydrogeologist

FIGURES