September 5, 2006

EnCana Oil & Gas (USA) Inc.

370 17th St., Suite 1700

Denver, CO 80202

Attn:  Mr. Chris Williams

Dear Mr. Williams:

Cordilleran Compliance Services, Inc. (Cordilleran) has been retained by EnCana Oil & Gas (USA) Inc. to perform technical environmental services including; quarterly groundwater and surface water assessment and remediation of groundwater that has been impacted by dissolved phase volatile hydrocarbons comprised primarily of methane and benzene in the area of the West Divide Creek Gas Seep (Figure 1).

### Executive Summary

The objective of the continued operation of the remediation system and quarterly water sampling is to mitigate and control migration of the dissolved phase hydrocarbons in the down gradient direction and into nearby Divide Creek; and surrounding area groundwater wells and to treat the source of the hydrocarbons.

This report summarizes sampling results of surface water and groundwater samples collected in June at the seep site in a continued effort to monitor the possible migration of the benzene plume prior to the COGCC staff rendering a decision on the reduction of the sampling frequency. Samples collected during this period were analyzed by Evergreen Analytical Laboratory (EAL) of Wheat Ridge, CO for BTEX/MTBE using EPA method 8021, and dissolved methane using method RSK 175M.

Analytes to be included in addition to BTEX/MTBE and dissolved methane in the September sampling event are: total dissolved solids (TDS), pH, specific conductance, chloride (Cl), sodium (Na), isotopic methane gas determination (where dissolved gasses is sufficient), and the existing list of field parameters (temperature, specific conductance, dissolved oxygen, pH, total dissolved solids, and turbidity)

Groundwater Status

Groundwater samples and field parameters were collected from June 12, 2006 through June 13, 2006 from monitoring wells (MW-1, 2, 4, 6-9, 11-27) (Figure 1). Prior to sample collection, static water levels were measured in monitoring wells to within 0.01 feet (ft) from the top of the PVC casing using an electronic water level indicator. Groundwater elevations are graphically illustrated in Figure 2. The hydraulic gradient was determined to be 3.1 x 10-2 ft/ft. A total of three casing volumes were removed prior to sampling each well using dedicated disposable bailers with bottom loading valve assemblies. Field parameters were obtained at the completion of purging activities (Table 1). Groundwater samples were collected following field parameter measurements. Groundwater samples were placed in the appropriate sample containers provided by EAL, labeled, stored on ice, and delivered under chain-of-custody procedures to EAL. Groundwater was analyzed for BTEX/MTBE and dissolved methane.

Benzene

Monitoring wells 2, 4, 9 and 12 have benzene concentrations of 280μgl, 220μgl, 8.6μgl and 1.2μgl respectively. Monitoring wells 2, 4 and 9 have benzene concentrations exceeding the Colorado Oil & Gas Conservation (COGCC) ground water standard of 5μgl. The laboratory results for benzene concentrations for each monitoring well are summarized in Table 2. Benzene concentrations are graphically illustrated in Figure 3. Benzene was not detected in monitoring wells 1, 6, 7, 8, 11, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26 and 27. The size of the area underlain by groundwater that is impacted by benzene at concentrations above 1.0μgl is approximately 89,428 ft2.

Toluene

Toluene was not detected in Divide Creek monitoring wells during the June quarter of 2006 (Table 3).

Ethylbenzene

Ethylbenzene was present in monitoring well 2 and monitoring well 4 at concentrations of 11μgl and 8.3μgl respectively. These concentrations are below the COGCC groundwater standard of 680μgl. Ethylbenzene was not detected in any other monitoring wells (Table 4).

Total Xylenes

Total Xylenes were present in monitoring wells 2, 4 and 9 at concentrations of 93μgl, 74μgl and 5.3μgl respectively. These concentrations are far below the COGCC basic groundwater standard of 10,000μgl. Total xylenes were not detected in the remaining monitoring wells. The laboratory results for total xylenes concentrations are summarized in Table 5.

MTBE

Laboratory results indicate that MTBE (methyl tertiary-butyl ether) was not present above the detectable limit in the groundwater samples collected in the monitoring area during the June quarter of 2006 (Table 6).

Dissolved Methane

Dissolved methane was detected in all monitoring wells except monitoring well 24 and monitoring well 27. Total dissolved methane is summarized in Table 7.  In general the highest concentrations of dissolved methane in the groundwater are located near the seep. Dissolved methane concentrations are graphically illustrated in Figure 4.

Surface-Water Quality

Cordilleran collected eight surface water samples (DCS-1-8) and field parameters (temperature, specific conductance, dissolved oxygen, pH, total dissolved solids, and turbidity) from the West Divide Creek stream (Table 8). Surface water samples were placed in the appropriate sample containers provided by EAL, labeled, stored on ice, and delivered under strict chain-of-custody procedures to EAL, Wheat Ridge, Colorado. Surface water was analyzed for BTEX/MTBE and dissolved methane.

Laboratory results indicate that BTEX/MTBE compounds were not detected in any of the Divide Creek surface water samples (Tables 9-13) Dissolved methane was detected in all eight samples; all with concentrations measuring less than 0.02 mgl (Table 14).

QA/QC

Laboratory and field quality assurance and quality control (QA/QC) consisted of analyzing duplicate samples, matrix spikes and duplicate analyses. For quality assurance, duplicate water samples were acquired at an approximate rate of 1 for every 10 samples. In addition replicate/split samples were also obtained at an approximate rate of 1 for every 10 samples. Replicate/split samples were sent to ESN Rocky Mountain, Golden, Colorado. During the June quarter of 2006 three duplicate and three replicate/split samples were obtained. Analytical results indicate insignificant differences between actual and replicate samples.

Divide Creek Seep Status

The air sparge remediation system has operated nearly continuously throughout the quarter. Since the start up of the system in April of 2004, the number of wells with benzene concentrations above the maximum contaminant level (MCL) in the area has been reduced to 3. The size of the area underlain by groundwater that is impacted by benzene at concentrations above 1.0μgl has been reduced from 134,974 ft2 in June of 2005 to approximately 89,428 ft2 in June of 2006, a total reduction of 45,545 ft2. Presently, all samples collected during the June 2006 located within and downgradient of the system do not have any benzene concentrations above the MCL. All wells with benzene concentrations above the MCL are located upgradient of the system. Benzene greater than 1.0μgl in the groundwater is primarily located within 300 feet of the seep. BTEX/MTBE was not detected in the surface water samples. Dissolved methane was not detected above 0.02 mg/l in the surface water samples. Dissolved methane in the groundwater greater than 1.0mgl is predominantly found within 200 feet of the seep. Dissolved oxygen readings have increased since the implementation of the remediation system. Therefore, the remediation system has been effectively mitigating the plume.

Cordilleran appreciates the opportunity to provide services to EnCana Oil & Gas (USA) Inc.  If you have any questions or concerns regarding this information, please contact our offices.

Sincerely,

Cordilleran Compliance Services, Inc.

Dion Plsek, P.E.

Principal Engineer

Enclosures

TABLES

| SiteID | Date | SampleID | pH, Field (SU) | SpCond, Field (mS/cm) | TDS, Field (g/L) | Temp, Field (°C) | Turbidity, Field (NTU) | DO, Field (mg/L) | DO (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MW01 | 12-Jun-06 | 061206-MW1 | 7.39 | 1.305 | 0.8 | 13.52 | 1089 | 1.29 | 15 |
| MW02 | 12-Jun-06 | 061206-MW2 | 7.47 | 0.733 | 0.5 | 14.49 | 1164 | 0.88 | 10.3 |
| MW04 | 12-Jun-06 | 061206-MW4 | 7.78 | 0.713 | 0.5 | 14 | 184 | 0.77 | 8.6 |
| MW06 | 12-Jun-06 | 061206-MW6 | 6.81 | 1.424 | 0.9 | 11.72 | 1082 | 2.16 | 24.2 |
| MW07 | 12-Jun-06 | 061206-MW7 | 7.41 | 1.189 | 0.8 | 12.66 | 1340 | 3.3 | 37.9 |
| MW08 | 12-Jun-06 | 061206-MW08 | 7.37 | 1.189 | 0.8 | 13.33 | 508 | 1.36 | 15.7 |
| MW09 | 12-Jun-06 | 061206-MW9 | 7.43 | 0.701 | 0.5 | 11.43 | 334 | 1.87 | 20.6 |
| MW11 | 12-Jun-06 | 061206-MW11 | 7.19 | 0.562 | 0.4 | 7.98 | 206 | 2.9 | 29.5 |
| MW12 | 12-Jun-06 | 061206-MW12 | 7.17 | 0.849 | 0.5 | 9.14 | 72.1 | 2.19 | 23.1 |
| MW13 | 12-Jun-06 | 061206-MW13 | 7.54 | 0.935 | 0.6 | 14.13 | 2000 | 1.86 | 21.7 |
| MW14 | 12-Jun-06 | 061206-MW14 | 7.21 | 0.643 | 0.4 | 11.32 | 2000 | 1.51 | 16.7 |
| MW15 | 12-Jun-06 | 061206-MW15 | 7.58 | 0.605 | 0.4 | 8.38 | 257 | 3.58 | 36.7 |
| MW16 | 12-Jun-06 | 061206-MW16 | 2.93 | 0.933 | 0.6 | 12.56 | 91.6 | 1.05 | 12.1 |
| MW17 | 12-Jun-06 | 061206-MW17 | 7.57 | 1.302 | 0.8 | 14.87 | 1093 | 1.28 | 15.4 |
| MW18 | 13-Jun-06 | 061306-MW18(Split) | 8.65 | 0.716 | 0.5 | 10.21 | 225 | 2.31 | 26.2 |
| MW18 | 13-Jun-06 | 061306-MW18 | 8.65 | 0.716 | 0.5 | 10.21 | 225 | 2.31 | 26.2 |
| MW18 | 13-Jun-06 | 061306-XY(Dup) | 8.65 | 0.716 | 0.5 | 10.21 | 225 | 2.31 | 26.2 |
| MW19 | 12-Jun-06 | 061206-MW19 | 7.19 | 0.688 | 0.4 | 13.02 | 33.3 | 1.23 | 13.8 |
| MW20 | 13-Jun-06 | 061306-MW20 | 8.42 | 1.85 | 0.3 | 8.97 | 653 | 2.24 | 25.1 |
| MW21 | 13-Jun-06 | 061306-MW21 | 8.81 | 1.149 | 0.7 | 9.55 | 908 | 2.21 | 24.7 |
| MW22 | 13-Jun-06 | 061306-MW22 | 8.42 | 1.208 | 0.8 | 8.8 | 939 | 2.33 | 25.5 |
| MW23 | 13-Jun-06 | 061306-MW23 | 8.86 | 1.468 | 0.9 | 9.48 | 2000 | 2.53 | 28.1 |
| MW24 | 13-Jun-06 | 061306-MW24 | 8.67 | 0.705 | 0.5 | 10.38 | 455 | 9.27 | 48.4 |
| MW25 | 12-Jun-06 | 061206-MW25 | 7.5 | 0.536 | 0.3 | 7.92 | 253 | 2.73 | 30 |
| MW26 | 13-Jun-06 | 061306-MW26(Split) | 8.57 | 0.786 | 0.5 | 10.37 | 5999 | 1.95 | 22.1 |
| MW26 | 13-Jun-06 | 061306-MW00(Dup) | 8.57 | 0.786 | 0.5 | 10.37 | 5999 | 1.95 | 22.1 |
| MW26 | 13-Jun-06 | 061306-MW26 | 8.57 | 0.786 | 0.5 | 10.37 | 5999 | 1.95 | 22.1 |
| MW27 | 13-Jun-06 | 061306-MW27 | 8.88 | 1.025 | 0.7 | 11.75 | 219 | 2.31 | 26.8 |

Table 1 (Divide Creek monitoring wells field parameters)

| SiteID | Date | SampleID | Benzene (µg/L) | BenzeneDL (µg/L) |
| --- | --- | --- | --- | --- |
| MW01 | 12-Jun-06 | 061206-MW1 | Non Detect | 1 |
| MW02 | 12-Jun-06 | 061206-MW2 | 280 | 5 |
| MW04 | 12-Jun-06 | 061206-MW4 | 220 | 1 |
| MW06 | 12-Jun-06 | 061206-MW6 | Non Detect | 1 |
| MW07 | 12-Jun-06 | 061206-MW7 | Non Detect | 1 |
| MW08 | 12-Jun-06 | 061206-MW08 | Non Detect | 1 |
| MW09 | 12-Jun-06 | 061206-MW9 | 8.6 | 1 |
| MW11 | 12-Jun-06 | 061206-MW11 | Non Detect | 1 |
| MW12 | 12-Jun-06 | 061206-MW12 | 1.2 | 1 |
| MW13 | 12-Jun-06 | 061206-MW13 | Non Detect | 1 |
| MW14 | 12-Jun-06 | 061206-MW14 | Non Detect | 1 |
| MW15 | 12-Jun-06 | 061206-MW15 | Non Detect | 1 |
| MW16 | 12-Jun-06 | 061206-MW16 | Non Detect | 1 |
| MW17 | 12-Jun-06 | 061206-MW17 | Non Detect | 1 |
| MW18 | 13-Jun-06 | 061306-MW18(Split) | Non Detect | 0.5 |
| MW18 | 13-Jun-06 | 061306-MW18 | Non Detect | 1 |
| MW18 | 13-Jun-06 | 061306-XY(Dup) | Non Detect | 1 |
| MW19 | 12-Jun-06 | 061206-MW19 | Non Detect | 1 |
| MW20 | 13-Jun-06 | 061306-MW20 | Non Detect | 1 |
| MW21 | 13-Jun-06 | 061306-MW21 | Non Detect | 1 |
| MW22 | 13-Jun-06 | 061306-MW22 | Non Detect | 1 |
| MW23 | 13-Jun-06 | 061306-MW23 | Non Detect | 1 |
| MW24 | 13-Jun-06 | 061306-MW24 | Non Detect | 1 |
| MW25 | 12-Jun-06 | 061206-MW25 | Non Detect | 1 |
| MW26 | 13-Jun-06 | 061306-MW26(Split) | Non Detect | 0.5 |
| MW26 | 13-Jun-06 | 061306-MW00(Dup) | Non Detect | 1 |
| MW26 | 13-Jun-06 | 061306-MW26 | Non Detect | 1 |
| MW27 | 13-Jun-06 | 061306-MW27 | Non Detect | 1 |

Table 2 (Divide Creek monitoring wells results for Benzene concentrations)

| SiteID | Date | SampleID | Toluene (µg/L) | TolueneDL (µg/L) |
| --- | --- | --- | --- | --- |
| MW01 | 12-Jun-06 | 061206-MW1 | Non Detect | 2 |
| MW02 | 12-Jun-06 | 061206-MW2 | Non Detect | 2 |
| MW04 | 12-Jun-06 | 061206-MW4 | Non Detect | 2 |
| MW06 | 12-Jun-06 | 061206-MW6 | Non Detect | 2 |
| MW07 | 12-Jun-06 | 061206-MW7 | Non Detect | 2 |
| MW08 | 12-Jun-06 | 061206-MW08 | Non Detect | 2 |
| MW09 | 12-Jun-06 | 061206-MW9 | Non Detect | 2 |
| MW11 | 12-Jun-06 | 061206-MW11 | Non Detect | 2 |
| MW12 | 12-Jun-06 | 061206-MW12 | Non Detect | 2 |
| MW13 | 12-Jun-06 | 061206-MW13 | Non Detect | 2 |
| MW14 | 12-Jun-06 | 061206-MW14 | Non Detect | 2 |
| MW15 | 12-Jun-06 | 061206-MW15 | Non Detect | 2 |
| MW16 | 12-Jun-06 | 061206-MW16 | Non Detect | 2 |
| MW17 | 12-Jun-06 | 061206-MW17 | Non Detect | 2 |
| MW18 | 13-Jun-06 | 061306-MW18(Split) | Non Detect | 0.5 |
| MW18 | 13-Jun-06 | 061306-MW18 | Non Detect | 2 |
| MW18 | 13-Jun-06 | 061306-XY(Dup) | Non Detect | 2 |
| MW19 | 12-Jun-06 | 061206-MW19 | Non Detect | 2 |
| MW20 | 13-Jun-06 | 061306-MW20 | Non Detect | 2 |
| MW21 | 13-Jun-06 | 061306-MW21 | Non Detect | 2 |
| MW22 | 13-Jun-06 | 061306-MW22 | Non Detect | 2 |
| MW23 | 13-Jun-06 | 061306-MW23 | Non Detect | 2 |
| MW24 | 13-Jun-06 | 061306-MW24 | Non Detect | 2 |
| MW25 | 12-Jun-06 | 061206-MW25 | Non Detect | 2 |
| MW26 | 13-Jun-06 | 061306-MW26(Split) | Non Detect | 0.5 |
| MW26 | 13-Jun-06 | 061306-MW00(Dup) | Non Detect | 2 |
| MW26 | 13-Jun-06 | 061306-MW26 | Non Detect | 2 |
| MW27 | 13-Jun-06 | 061306-MW27 | Non Detect | 2 |

Table 3 (Divide Creek monitoring wells results for Toluene concentrations)

| SiteID | Date | SampleID | Ethylbenzene (µg/L) | EthylbenzeneDL(µg/L) |
| --- | --- | --- | --- | --- |
| MW01 | 12-Jun-06 | 061206-MW1 | Non Detect | 2 |
| MW02 | 12-Jun-06 | 061206-MW2 | 11 | 2 |
| MW04 | 12-Jun-06 | 061206-MW4 | 8.3 | 2 |
| MW06 | 12-Jun-06 | 061206-MW6 | Non Detect | 2 |
| MW07 | 12-Jun-06 | 061206-MW7 | Non Detect | 2 |
| MW08 | 12-Jun-06 | 061206-MW08 | Non Detect | 2 |
| MW09 | 12-Jun-06 | 061206-MW9 | Non Detect | 2 |
| MW11 | 12-Jun-06 | 061206-MW11 | Non Detect | 2 |
| MW12 | 12-Jun-06 | 061206-MW12 | Non Detect | 2 |
| MW13 | 12-Jun-06 | 061206-MW13 | Non Detect | 2 |
| MW14 | 12-Jun-06 | 061206-MW14 | Non Detect | 2 |
| MW15 | 12-Jun-06 | 061206-MW15 | Non Detect | 2 |
| MW16 | 12-Jun-06 | 061206-MW16 | Non Detect | 2 |
| MW17 | 12-Jun-06 | 061206-MW17 | Non Detect | 2 |
| MW18 | 13-Jun-06 | 061306-MW18(Split) | Non Detect | 0.5 |
| MW18 | 13-Jun-06 | 061306-MW18 | Non Detect | 2 |
| MW18 | 13-Jun-06 | 061306-XY(Dup) | Non Detect | 2 |
| MW19 | 12-Jun-06 | 061206-MW19 | Non Detect | 2 |
| MW20 | 13-Jun-06 | 061306-MW20 | Non Detect | 2 |
| MW21 | 13-Jun-06 | 061306-MW21 | Non Detect | 2 |
| MW22 | 13-Jun-06 | 061306-MW22 | Non Detect | 2 |
| MW23 | 13-Jun-06 | 061306-MW23 | Non Detect | 2 |
| MW24 | 13-Jun-06 | 061306-MW24 | Non Detect | 2 |
| MW25 | 12-Jun-06 | 061206-MW25 | Non Detect | 2 |
| MW26 | 13-Jun-06 | 061306-MW26(Split) | Non Detect | 0.5 |
| MW26 | 13-Jun-06 | 061306-MW00(Dup) | Non Detect | 2 |
| MW26 | 13-Jun-06 | 061306-MW26 | Non Detect | 2 |
| MW27 | 13-Jun-06 | 061306-MW27 | Non Detect | 2 |

Table 4 (Divide Creek monitoring wells results for Ethylbenzene concentrations)

| SiteID | Date | SampleID | mp-Xylene (µg/L) | mp-XyleneDL (µg/L) | o-Xylene (µg/L) | o-XyleneDL (µg/L) |
| --- | --- | --- | --- | --- | --- | --- |
| MW01 | 12-Jun-06 | 061206-MW1 | Non Detect | 2 | Non Detect | 2 |
| MW02 | 12-Jun-06 | 061206-MW2 | 81 | 2 | 12 | 2 |
| MW04 | 12-Jun-06 | 061206-MW4 | 63 | 2 | 11 | 2 |
| MW06 | 12-Jun-06 | 061206-MW6 | Non Detect | 2 | Non Detect | 2 |
| MW07 | 12-Jun-06 | 061206-MW7 | Non Detect | 2 | Non Detect | 2 |
| MW08 | 12-Jun-06 | 061206-MW08 | Non Detect | 2 | Non Detect | 2 |
| MW09 | 12-Jun-06 | 061206-MW9 | 5.3 | 2 | Non Detect | 2 |
| MW11 | 12-Jun-06 | 061206-MW11 | Non Detect | 2 | Non Detect | 2 |
| MW12 | 12-Jun-06 | 061206-MW12 | Non Detect | 2 | Non Detect | 2 |
| MW13 | 12-Jun-06 | 061206-MW13 | Non Detect | 2 | Non Detect | 2 |
| MW14 | 12-Jun-06 | 061206-MW14 | Non Detect | 2 | Non Detect | 2 |
| MW15 | 12-Jun-06 | 061206-MW15 | Non Detect | 2 | Non Detect | 2 |
| MW16 | 12-Jun-06 | 061206-MW16 | Non Detect | 2 | Non Detect | 2 |
| MW17 | 12-Jun-06 | 061206-MW17 | Non Detect | 2 | Non Detect | 2 |
| MW18 | 13-Jun-06 | 061306-MW18(Split) | Non Detect | 1 | Non Detect | 0.5 |
| MW18 | 13-Jun-06 | 061306-MW18 | Non Detect | 2 | Non Detect | 2 |
| MW18 | 13-Jun-06 | 061306-XY(Dup) | Non Detect | 2 | Non Detect | 2 |
| MW19 | 12-Jun-06 | 061206-MW19 | Non Detect | 2 | Non Detect | 2 |
| MW20 | 13-Jun-06 | 061306-MW20 | Non Detect | 2 | Non Detect | 2 |
| MW21 | 13-Jun-06 | 061306-MW21 | Non Detect | 2 | Non Detect | 2 |
| MW22 | 13-Jun-06 | 061306-MW22 | Non Detect | 2 | Non Detect | 2 |
| MW23 | 13-Jun-06 | 061306-MW23 | Non Detect | 2 | Non Detect | 2 |
| MW24 | 13-Jun-06 | 061306-MW24 | Non Detect | 2 | Non Detect | 2 |
| MW25 | 12-Jun-06 | 061206-MW25 | Non Detect | 2 | Non Detect | 2 |
| MW26 | 13-Jun-06 | 061306-MW26(Split) | Non Detect | 1 | Non Detect | 0.5 |
| MW26 | 13-Jun-06 | 061306-MW00(Dup) | Non Detect | 2 | Non Detect | 2 |
| MW26 | 13-Jun-06 | 061306-MW26 | Non Detect | 2 | Non Detect | 2 |
| MW27 | 13-Jun-06 | 061306-MW27 | Non Detect | 2 | Non Detect | 2 |

Table 5 (Divide Creek monitoring wells results for Total Xylenes concentrations)

| SiteID | Date | SampleID | MTBE (µg/L) | MTBE-DL (µg/L) |
| --- | --- | --- | --- | --- |
| MW01 | 12-Jun-06 | 061206-MW1 | Not Detect | 4 |
| MW02 | 12-Jun-06 | 061206-MW2 | Not Detect | 4 |
| MW04 | 12-Jun-06 | 061206-MW4 | Not Detect | 4 |
| MW06 | 12-Jun-06 | 061206-MW6 | Not Detect | 4 |
| MW07 | 12-Jun-06 | 061206-MW7 | Not Detect | 4 |
| MW08 | 12-Jun-06 | 061206-MW08 | Not Detect | 4 |
| MW09 | 12-Jun-06 | 061206-MW9 | Not Detect | 4 |
| MW11 | 12-Jun-06 | 061206-MW11 | Not Detect | 4 |
| MW12 | 12-Jun-06 | 061206-MW12 | Not Detect | 4 |
| MW13 | 12-Jun-06 | 061206-MW13 | Not Detect | 4 |
| MW14 | 12-Jun-06 | 061206-MW14 | Not Detect | 4 |
| MW15 | 12-Jun-06 | 061206-MW15 | Not Detect | 4 |
| MW16 | 12-Jun-06 | 061206-MW16 | Not Detect | 4 |
| MW17 | 12-Jun-06 | 061206-MW17 | Not Detect | 4 |
| MW18 | 13-Jun-06 | 061306-MW18(Split) | Not Detect | 0.5 |
| MW18 | 13-Jun-06 | 061306-MW18 | Not Detect | 4 |
| MW18 | 13-Jun-06 | 061306-XY(Dup) | Not Detect | 4 |
| MW19 | 12-Jun-06 | 061206-MW19 | Not Detect | 4 |
| MW20 | 13-Jun-06 | 061306-MW20 | Not Detect | 4 |
| MW21 | 13-Jun-06 | 061306-MW21 | Not Detect | 4 |
| MW22 | 13-Jun-06 | 061306-MW22 | Not Detect | 4 |
| MW23 | 13-Jun-06 | 061306-MW23 | Not Detect | 4 |
| MW24 | 13-Jun-06 | 061306-MW24 | Not Detect | 4 |
| MW25 | 12-Jun-06 | 061206-MW25 | Not Detect | 4 |
| MW26 | 13-Jun-06 | 061306-MW26(Split) | Not Detect | 0.5 |
| MW26 | 13-Jun-06 | 061306-MW00(Dup) | Not Detect | 4 |
| MW26 | 13-Jun-06 | 061306-MW26 | Not Detect | 4 |
| MW27 | 13-Jun-06 | 061306-MW27 | Not Detect | 4 |

Table 6 (Divide Creek monitoring wells results for MTBE concentrations)

| SiteID | Date | SampleID | Methane (mg/L) | methaneDL (mg/L) |
| --- | --- | --- | --- | --- |
| MW01 | 12-Jun-06 | 061206-MW1 | 0.011 | 0.0008 |
| MW02 | 12-Jun-06 | 061206-MW2 | 12 | 0.04 |
| MW04 | 12-Jun-06 | 061206-MW4 | 9.2 | 0.04 |
| MW06 | 12-Jun-06 | 061206-MW6 | 0.0026 | 0.0008 |
| MW07 | 12-Jun-06 | 061206-MW7 | 0.0037 | 0.0008 |
| MW08 | 12-Jun-06 | 061206-MW08 | 0.13 | 0.0008 |
| MW09 | 12-Jun-06 | 061206-MW9 | 10 | 0.04 |
| MW11 | 12-Jun-06 | 061206-MW11 | 0.043 | 0.0008 |
| MW12 | 12-Jun-06 | 061206-MW12 | 2.1 | 0.016 |
| MW13 | 12-Jun-06 | 061206-MW13 | 2.8 | 0.016 |
| MW14 | 12-Jun-06 | 061206-MW14 | 7 | 0.04 |
| MW15 | 12-Jun-06 | 061206-MW15 | 0.01 | 0.0008 |
| MW16 | 12-Jun-06 | 061206-MW16 | 0.042 | 0.0008 |
| MW17 | 12-Jun-06 | 061206-MW17 | 2.6 | 0.016 |
| MW18 | 13-Jun-06 | 061306-MW18(Split) | 1.46 | 0.000068 |
| MW18 | 13-Jun-06 | 061306-MW18 | 1.4 | 0.016 |
| MW18 | 13-Jun-06 | 061306-XY(Dup) | 1.2 | 0.016 |
| MW19 | 12-Jun-06 | 061206-MW19 | 1.3 | 0.016 |
| MW20 | 13-Jun-06 | 061306-MW20 | 0.00855 | 0.0008 |
| MW21 | 13-Jun-06 | 061306-MW21 | 0.0032 | 0.0008 |
| MW22 | 13-Jun-06 | 061306-MW22 | 0.0014 | 0.0008 |
| MW23 | 13-Jun-06 | 061306-MW23 | 0.92 | 0.016 |
| MW24 | 13-Jun-06 | 061306-MW24 | Non Detect | 0.0008 |
| MW25 | 12-Jun-06 | 061206-MW25 | 0.06 | 0.0008 |
| MW26 | 13-Jun-06 | 061306-MW26(Split) | 0.767 | 0.000068 |
| MW26 | 13-Jun-06 | 061306-MW00(Dup) | 0.74 | 0.008 |
| MW26 | 13-Jun-06 | 061306-MW26 | 0.63 | 0.008 |
| MW27 | 13-Jun-06 | 061306-MW27 | Non Detect | 0.0008 |

Table 7 (Divide Creek monitoring wells results for Dissolved Methane concentrations)

| SiteID | Date | SampleID | pH, Field (SU) | SpCond, Field (mS/cm) | TDS, Field (g/L) | Temp, Field (°C) | Turbidity, Field (NTU) | DO, Field (mg/L) | DO (%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DCS1 | 13-Jun-06 | 061306-DCS1 | 8.63 | 0.413 | 0.3 | 15.35 | 7.6 | 8.53 | 103.8 |
| DCS2 | 13-Jun-06 | 061306-DCS2 | 8.54 | 0.43 | 0.3 | 15.09 | 56.6 | 7.9 | 95.6 |
| DCS3 | 13-Jun-06 | 061306-DCS3 | 8.68 | 0.415 | 0.3 | 15.61 | 46.3 | 8.05 | 98.9 |
| DCS4 | 13-Jun-06 | 061306-DCS4 | 8.64 | 0.414 | 0.3 | 15.78 | 113 | 8.16 | 100.3 |
| DCS5 | 13-Jun-06 | 061306-DCS5 | 8.7 | 0.417 | 0.3 | 15.91 | 42.7 | 7.97 | 98.1 |
| DCS5 | 13-Jun-06 | 061306-DCS5(Split) | 8.7 | 0.417 | 0.3 | 15.91 | 42.7 | 7.97 | 98.1 |
| DCS6 | 13-Jun-06 | 061306-DCS6 | 8.75 | 0.42 | 0.3 | 16.01 | 51.7 | 8 | 98.8 |
| DCS7 | 13-Jun-06 | 061306-DCS7 | 8.72 | 0.42 | 0.3 | 16.13 | 40.9 | 7.91 | 97.9 |
| DCS8 | 13-Jun-06 | 061306-DCS8 | 8.74 | 0.42 | 0.3 | 16.33 | 42.3 | 7.83 | 97.4 |
| DCS8 | 13-Jun-06 | 061306-YZ(Dup) | 8.74 | 0.42 | 0.3 | 16.33 | 42.3 | 7.83 | 97.4 |

Table 8 (Divide Creek surface water results for Dissolved Methane concentrations)

| SiteID | Date | SampleID | Benzene (µg/L) | BenzeneDL (µg/L) |
| --- | --- | --- | --- | --- |
| DCS1 | 13-Jun-06 | 061306-DCS1 | Non Detect | 1 |
| DCS2 | 13-Jun-06 | 061306-DCS2 | Non Detect | 1 |
| DCS3 | 13-Jun-06 | 061306-DCS3 | Non Detect | 1 |
| DCS4 | 13-Jun-06 | 061306-DCS4 | Non Detect | 1 |
| DCS5 | 13-Jun-06 | 061306-DCS5 | Non Detect | 1 |
| DCS5 | 13-Jun-06 | 061306-DCS5(Split) | Non Detect | 0.5 |
| DCS6 | 13-Jun-06 | 061306-DCS6 | Non Detect | 1 |
| DCS7 | 13-Jun-06 | 061306-DCS7 | Non Detect | 1 |
| DCS8 | 13-Jun-06 | 061306-DCS8 | Non Detect | 1 |
| DCS8 | 13-Jun-06 | 061306-YZ(Dup) | Non Detect | 1 |

Table 9 (Divide Creek Surface water results Benzene concentrations)

| SiteID | Date | SampleID | Toluene (µg/L) | TolueneDL( µg/L) |
| --- | --- | --- | --- | --- |
| DCS1 | 13-Jun-06 | 061306-DCS1 | Non Detect | 2 |
| DCS2 | 13-Jun-06 | 061306-DCS2 | Non Detect | 2 |
| DCS3 | 13-Jun-06 | 061306-DCS3 | Non Detect | 2 |
| DCS4 | 13-Jun-06 | 061306-DCS4 | Non Detect | 2 |
| DCS5 | 13-Jun-06 | 061306-DCS5 | Non Detect | 2 |
| DCS5 | 13-Jun-06 | 061306-DCS5(Split) | Non Detect | 0.5 |
| DCS6 | 13-Jun-06 | 061306-DCS6 | Non Detect | 2 |
| DCS7 | 13-Jun-06 | 061306-DCS7 | Non Detect | 2 |
| DCS8 | 13-Jun-06 | 061306-DCS8 | Non Detect | 2 |
| DCS8 | 13-Jun-06 | 061306-YZ(Dup) | Non Detect | 2 |

Table 10 (Divide Creek Surface water results for Toluene concentrations)

| SiteID | Date | SampleID | Ethylbenzene (µg/L) | EthylbenzeneDL (µg/L) |
| --- | --- | --- | --- | --- |
| DCS1 | 13-Jun-06 | 061306-DCS1 | Non Detect | 2 |
| DCS2 | 13-Jun-06 | 061306-DCS2 | Non Detect | 2 |
| DCS3 | 13-Jun-06 | 061306-DCS3 | Non Detect | 2 |
| DCS4 | 13-Jun-06 | 061306-DCS4 | Non Detect | 2 |
| DCS5 | 13-Jun-06 | 061306-DCS5 | Non Detect | 2 |
| DCS5 | 13-Jun-06 | 061306-DCS5(Split) | Non Detect | 0.5 |
| DCS6 | 13-Jun-06 | 061306-DCS6 | Non Detect | 2 |
| DCS7 | 13-Jun-06 | 061306-DCS7 | Non Detect | 2 |
| DCS8 | 13-Jun-06 | 061306-DCS8 | Non Detect | 2 |
| DCS8 | 13-Jun-06 | 061306-YZ(Dup) | Non Detect | 2 |

Table 11 (Divide Creek surface water results for Ethylbenzene concentrations)

| SiteID | Date | SampleID | mp-Xylene (µg/L) | mp-XyleneDL (µg/L) | o-Xylene (µg/L) | o-XyleneDL (µg/L) |
| --- | --- | --- | --- | --- | --- | --- |
| DCS1 | 13-Jun-06 | 061306-DCS1 | Non Detect | 2 | Non Detect | 2 |
| DCS2 | 13-Jun-06 | 061306-DCS2 | Non Detect | 2 | Non Detect | 2 |
| DCS3 | 13-Jun-06 | 061306-DCS3 | Non Detect | 2 | Non Detect | 2 |
| DCS4 | 13-Jun-06 | 061306-DCS4 | Non Detect | 2 | Non Detect | 2 |
| DCS5 | 13-Jun-06 | 061306-DCS5 | Non Detect | 2 | Non Detect | 2 |
| DCS5 | 13-Jun-06 | 061306-DCS5(Split) | Non Detect | 1 | Non Detect | 0.5 |
| DCS6 | 13-Jun-06 | 061306-DCS6 | Non Detect | 2 | Non Detect | 2 |
| DCS7 | 13-Jun-06 | 061306-DCS7 | Non Detect | 2 | Non Detect | 2 |
| DCS8 | 13-Jun-06 | 061306-DCS8 | Non Detect | 2 | Non Detect | 2 |
| DCS8 | 13-Jun-06 | 061306-YZ(Dup) | Non Detect | 2 | Non Detect | 2 |

Table 12 (Divide Creek surface water results for Total Xylenes concentrations)

| SiteID | Date | SampleID | MTBE (µg/L) | MTBE-DL (µg/L) |
| --- | --- | --- | --- | --- |
| DCS1 | 13-Jun-06 | 061306-DCS1 | Non Detect | 4 |
| DCS2 | 13-Jun-06 | 061306-DCS2 | Non Detect | 4 |
| DCS3 | 13-Jun-06 | 061306-DCS3 | Non Detect | 4 |
| DCS4 | 13-Jun-06 | 061306-DCS4 | Non Detect | 4 |
| DCS5 | 13-Jun-06 | 061306-DCS5 | Non Detect | 4 |
| DCS5 | 13-Jun-06 | 061306-DCS5(Split) | Non Detect | 0.5 |
| DCS6 | 13-Jun-06 | 061306-DCS6 | Non Detect | 4 |
| DCS7 | 13-Jun-06 | 061306-DCS7 | Non Detect | 4 |
| DCS8 | 13-Jun-06 | 061306-DCS8 | Non Detect | 4 |
| DCS8 | 13-Jun-06 | 061306-YZ(Dup) | Non Detect | 4 |

Table 13 (Divide Creek surface water results for MTBE concentrations)

| SiteID | Date | SampleID | Methane (mg/L) | MethaneDL (mg/L) |
| --- | --- | --- | --- | --- |
| DCS1 | 13-Jun-06 | 061306-DCS1 | 0.0016 | 0.0008 |
| DCS2 | 13-Jun-06 | 061306-DCS2 | 0.019 | 0.0008 |
| DCS3 | 13-Jun-06 | 061306-DCS3 | 0.005 | 0.0008 |
| DCS4 | 13-Jun-06 | 061306-DCS4 | 0.0038 | 0.0008 |
| DCS5 | 13-Jun-06 | 061306-DCS5 | 0.0045 | 0.0008 |
| DCS5 | 13-Jun-06 | 061306-DCS5(Split) | 0.00445 | 0.000068 |
| DCS6 | 13-Jun-06 | 061306-DCS6 | 0.0054 | 0.0008 |
| DCS7 | 13-Jun-06 | 061306-DCS7 | 0.0053 | 0.0008 |
| DCS8 | 13-Jun-06 | 061306-DCS8 | 0.0043 | 0.0008 |
| DCS8 | 13-Jun-06 | 061306-YZ(Dup) | 0.0048 | 0.0008 |

Table 14 (Divide Creek surface water results for Dissolved Methane concentrations)

FIGURES