April 30, 2004

Mr. Pepi Langegger

3807 Lupine Drive

Vail, Colorado 81657

RE:	Water Quality Analytical Results for Stream and Pond Samples

Section 12 – Township 7 South – Range 92 West

Garfield County, Colorado

West Divide Creek Gas Seep Investigation

COGCC Investigation No. 1802

Dear Mr. Langegger:

As discussed in my April 26, 2004 letter, the Colorado Oil and Gas Conservation Commission (COGCC) had not yet received the final analytical results from Evergreen Analytical Laboratory (Evergreen) for the April 8, 2004 sampling of West Divide Creek and stock ponds on your property.  We received the final analytical results on April 29, 2004 and this letter presents the results for Total Dissolved Solids (TDS) and a revision of the results for sodium (Na) which was previously reported.  The preliminary report for sodium concentrations identified laboratory induced blank contamination; however, the magnitude of the blank contamination was not included in the preliminary report.  This letter corrects the sodium concentration result for the blank contamination.  As with my previous letter, the analytical results from the water samples have been compared to applicable ground water and/or drinking water standards and are summarized below.  These water standards were established for public drinking water supplies and often people use and consume ground water from private wells and springs that can exceed these standards.

The sample locations are identified by the following names:

- West Divide Creek – Upstream (of the gas seep)

- Langegger Upper Stock Pond

- Langegger Lower Stock Pond

- Beaver Pond; West Divide Creek – Downstream (of the gas seep)

Total Dissolved Solids (TDS):  CDPHE has established a TDS standard for human drinking water of 500 milligrams per liter (mg/l).  Although CDPHE does not have an agricultural standard for TDS, other agencies recommend concentrations below 2,000 mg/l for irrigation, and below 5,000 mg/l for most livestock watering.  TDS concentrations are related to the presence of naturally occurring elements and chemical compounds such as chloride, sodium, potassium, calcium, magnesium, and sulfate.

West Divide Creek – Upstream

##### TDS was detected in the upstream water sample at a concentration of 245 mg/l, which is within the

CDPHE human drinking water standard, below the recommended maximum concentration for irrigation, and below the recommended maximum concentration for most livestock watering.

Langegger Upper Stock Pond

##### TDS was detected in the upstream water sample at a concentration of 383 mg/l, which is within the

CDPHE human drinking water standard, below the recommended maximum concentration for irrigation, and below the recommended maximum concentration for most livestock watering.

Langegger Lower Stock Pond

##### TDS was detected in the upstream water sample at a concentration of 385 mg/l, which is within the

CDPHE human drinking water standard, below the recommended maximum concentration for irrigation, and below the recommended maximum concentration for most livestock watering.

Beaver Pond; West Divide Creek – Downstream

##### TDS was detected in the upstream water sample at a concentration of 279 mg/l, which is within the

CDPHE human drinking water standard, below the recommended maximum concentration for irrigation, and below the recommended maximum concentration for most livestock watering.

Sodium (Na):  Although CDPHE does not have a standard for sodium, people on salt restricted diets should be aware of the Na concentration in the water they drink.  A concentration of drinking water with a concentration of sodium less than 20 mg/l is recommended by some for people on salt restricted diets or for people suffering from hypertension or heart disease.   Sodium occurs naturally in the ground water in many areas at concentrations that exceed the recommended level.

West Divide Creek – Upstream

Sodium was detected in the upstream water sample at a concentration of 29 “B” mg/l, which is slightly greater than the recommended level for people on salt restricted diets. The B notation is a laboratory “flag” which is used to indicate the presence of a laboratory-induced contaminant in your sample (known as blank [“B”] contamination).  In this case the laboratory noticed the presence of sodium in the analytical method control sample (called a method blank).  Normally the method blank does not have any compounds.  The actual concentration of sodium in your water well sample would be 29 mg/l less the blank detection (0.3182 mg/l) or 28.682 mg/l.

Langegger Upper Stock Pond

Sodium was detected in the water sample from your upper stock pond at a concentration of 36 “B” mg/l, which is greater than the recommended level for people on salt restricted diets.  The actual concentration of sodium in your water well sample would be 36 mg/l less the blank detection (0.3182 mg/l) or 35.682 mg/l.

Langegger Lower Stock Pond

Sodium was detected in the water sample from your lower stock pond at a concentration of 55 “B” mg/l, which is greater than the recommended level for people on salt restricted diets. The actual concentration of sodium in your water well sample would be 55 mg/l less the blank detection (0.3182 mg/l) or 54.682 mg/l.

Beaver Pond; West Divide Creek – Downstream

Sodium was detected in the water sample from your well at a concentration of 36 “B” mg/l, which is greater than the recommended level for people on salt restricted diets. The actual concentration of sodium in your water well sample would be 36 mg/l less the blank detection (0.3182 mg/l) or 35.682 mg/l.

If you have any questions or would like to discuss these matters further, please contact me or Debbie Baldwin at the COGCC in Denver via e-mail (robert.chesson@state.co.us); Debbie.baldwin@state.co.us) or by phone at 303-894-2100, extension 112 (Bob) or 111 (Debbie).

Respectfully,

Robert H. Chesson, C.P.G., P.G.

Environmental Protection Specialist

Enclosure

cc:	Rich Griebling – COGCC w/o enclosure

Debbie Baldwin – COGCC w/o enclosure

Jamie Adkins – COGCC w/o enclosure

Brian Macke – COGCC w/o enclosure