April 26, 2004

Mr. Jim Eubanks

6534 County Road 331

Silt, Colorado 81652

RE:	Water Quality Analytical Results for Your Water Well

Section 11 – Township 7 South – Range 92 West

Garfield County, Colorado

West Divide Creek Gas Seep Investigation

COGCC Investigation No. 1802

Dear Mr. Eubanks:

As you are aware, on April 8, 2004, Debbie Baldwin of the Colorado Oil and Gas Conservation Commission (COGCC) sampled your water well for evidence of possible impact from a natural gas production well release near your residence.  This release is the likely cause of the gas seep recently developed in West Divide Creek near your residence.   The COGCC has received the results for some of the analyses conducted on your well water and this letter presents these results.  The water well samples from your well were submitted to Evergreen Analytical Laboratory (Evergreen) in Wheat Ridge, Colorado, for analysis of inorganic chemical constituents, organic compounds associated with petroleum hydrocarbons, and methane gas.  At this time we have received analytical data for BTEX (an acronym for the organic chemical compounds benzene, ethylbenzene, toluene, and total xylenes), dissolved methane gas, and other inorganic chemical constituents.  We anticipate receipt of the remaining analytical tests in the very near future and will send you those results as soon as they are received. Copies of the initial analytical reports are enclosed. These fax copies are considered by the laboratory as preliminary; however, the concentrations reported rarely require any correction for final submittal and we do not expect any significant concentration changes with the final laboratory report.

The Water Quality Control Commission (WQCC) of the Colorado Department of Public Health and Environment (CDPHE) has established drinking water standards for the protection of human health. The analytical results from the water samples from your well have been compared to applicable ground water and/or drinking water standards and are summarized below.  Please keep in mind that these water standards were established for public drinking water supplies.  Often people use and consume ground water from private wells that can exceed these standards.

### COMPARISON OF INORGANIC ANALYTICAL RESULTS TO STANDARDS

(Please see enclosed Evergreen Analytical Laboratory Report)

- Sodium (Na):  Although CDPHE does not have a standard for sodium, people on salt restricted diets should be aware of the Na concentration in the water they drink.  A concentration of drinking water with a concentration of sodium less than 20 mg/l is recommended by some for people on salt restricted diets or for people suffering from hypertension or heart disease.   Sodium occurs naturally in the ground water in many areas at concentrations that exceed the recommended level.

Sodium was detected in the water sample from your well at a concentration of 360 “B” mg/l, which is greater than the recommended level for people on salt restricted diets. The B notation is a laboratory “flag” which is used to indicate the presence of a laboratory-induced contaminant in your sample (known as blank [“B”] contamination).  In this case the laboratory noticed the presence of sodium in the analytical method control sample (called a method blank).  Normally the method blank does not have any compounds.  Because this analysis result is preliminary, the laboratory has not yet sent the laboratory analysis quality control summary that discusses the magnitude of the blank contamination.

- Fluoride (F):  CDPHE has established a fluoride (F) standard for human drinking water is 4.0 mg/l.  Where fluoride concentrations are in the range of 0.7 mg/l to 1.2 mg/l, health benefits such as reduced dental decay have been observed.  Consumption of fluoride at concentrations of greater than 2.0 mg/l can result in mottling of teeth.  Consumption of fluoride at concentrations greater than 4.0 mg/l can increase the risk of skeletal fluorosis or other adverse health effects. 

Fluoride was detected in the water sample from your well at a concentration of 2.6 mg/l, which is within the human drinking water standard.

- Chloride (Cl):  The CDPHE chloride standard for drinking water is 250 mg/l.   Chloride concentrations in excess of 250 mg/l usually produce a noticeable taste in drinking water.

Chloride was detected in the water sample from your well at a concentration of 22.4 mg/l, which is below the CDPHE drinking water standard.

- Sulfate (SO4):  The CDPHE sulfate standard for drinking water is 250 mg/l.  Although CDPHE does not have an agricultural standard for sulfate, other agencies recommend a concentration below 1,500 mg/l for livestock watering.  Waters containing high concentrations of sulfate, typically caused by the leaching of natural deposits of magnesium sulfate (Epsom salts) or sodium sulfate (Glauber's salt), may be undesirable because of their laxative effects. Sulfate occurs naturally in the ground water in many areas in Colorado at concentrations that exceed the drinking water standard.

Sulfate was detected in the water sample from your well at a concentration of 361 mg/l, which is greater than the CDPHE drinking water standard and below the recommended maximum concentration for most livestock watering.

- Total Nitrate (NO3) + Nitrite (NO2) as Nitrogen (N):  The CDPHE total nitrate (NO3) + nitrite (NO2) as nitrogen (N) for standard for human drinking water is 10 mg/l.  Nitrate and nitrite are common contaminants in ground water from agricultural sources, such as fertilizer and animal, including human, wastes.  They are known to cause infant cyanosis or “blue baby disease” in humans and, at concentrations greater than 100 mg/l as nitrogen (N), may be dangerous to livestock.  High concentrations of nitrate and nitrite in ground water are known to occur in agricultural areas in Colorado.  

Total nitrate/nitrite, as N was not detected in the sample from your well.

- Iron (Fe):  The CDPHE standard for human drinking water for iron is 0.3 mg/l.  Small amounts of iron are common in ground water.  Iron may produce a brownish-red color in laundered clothing, can leave reddish stains on fixtures, and impart a metallic taste to beverages and food made with it.  After a period of time iron deposits can build up in pressure tanks, water heaters, and pipelines, reducing the effective flow rate and efficiency of the water supply.

Iron was not detected in the water sample from your well.

- Selenium (Se): The CDPHE selenium standard for human drinking water is 0.05 mg/l and the agricultural standard is 0.02 mg/l.  Excessive selenium (Se) (concentrations greater than 0.05 mg/l) can cause loss of hair and/or fingernails as well as adverse effects on the central nervous system.  Selenium (Se) occurs naturally in the ground water in many areas of Colorado at concentrations that exceed the drinking water standard.

Selenium was not detected in the water sample from your well.

- Calcium (Ca), Potassium (K), Magnesium (Mg), Bicarbonate (HCO3) and Carbonate (CO3) were also tested for in your water.  There are no standards from CDPHE for these parameters.

ORGANIC COMPOUNDS ASSOCIATED WITH PETROLEUM HYDROCARBONS

(please see enclosed Evergreen Analytical METHOD E 602/SW 8021B, BTEX DATA REPORT)

- Benzene: CDPHE's basic ground water standard for benzene is 5 micrograms per liter (µg/l).  Benzene was not detected in the sample from your water well.

- Toluene: CDPHE's basic ground water standard for toluene is 1,000 µg/l.  Toluene was not detected in the sample from your water well.

- Ethylbenzene: CDPHE's basic ground water standard for ethylbenzene is 680 µg/l.  Ethylbenzene was not detected in the sample from your water well.

- Total Xylenes (sum of m,p, and o-xylene): CDPHE's basic ground water standard for total xylenes is 10,000 µg/l.  Total xylenes were not detected in the sample from your water well.

METHANE GAS CONCENTRATION

(Please see enclosed Evergreen Analytical Laboratory METHANE REPORT)

- Methane was detected in the sample from your water well at 0.058 mg/l.

Methane gas alone is physiologically inert and non-toxic to humans.  Normal breath exhalation contains 1 to 99 ppm of methane (parts per million [ppm] is the same units as mg/l).  The presence of methane in drinking water does not present a known health hazard to humans or other animals via ingestion; however, methane in domestic water supplies can be associated with undesirable and potentially serious side effects.

Methane gas dissolved in water “exsolves” when exposed to the atmosphere and dissipates rapidly because it is lighter than air.  This is often responsible for the “fizzing” observed in water wells that may contain methane gas.  If the methane occurs at a high enough concentration and if it is allowed to accumulate in a confined space, such as a well pit, crawl space, closet, etc., an explosion hazard can be established.  In addition, if methane concentrations in well water are high, then pockets of free gas form within the water and cause the well pump to cavitate and no longer bring water to the surface.

Methane gas is common in water wells in Colorado.  It occurs naturally and the source of the methane is commonly from one or more of the sources listed below.

- Methane is commonly found as a gas in coal or black (organic rich) shale seams in the subsurface.

- Methane is a byproduct of the decay of organic matter.

- As the result of extensive testing for methane gas in water wells throughout Colorado, concentrations of methane gas below 1 mg/l are generally considered harmless, with concern for possible hazards from the methane increasing at concentration levels in well water at 7 mg/l and higher.  When methane levels are 13 mg/l or above, treatment is highly recommended.  Treatment generally includes some form of aeration to remove the methane from well water and ventilation to allow the methane to safely dissipate outdoors rather than accumulate within your home, well house or other confined space.

CONCLUSION

Because your water exceeded the CDPHE drinking water health advisory for  sodium (Na) and sulfate (SO4), and because you or your livestock and/or pets drink your water, then you may wish to discuss the possible health effects of continued consumption with your physician and/or veterinarian.  Information regarding possible health effects of consumption of water from your well is also available from Ms. Mary Meisner, the Garfield County Public Health Nurse/Public Health Officer.  Ms. Mesiner has an office in Rifle, Colorado (902 Taughenbaugh Blvd, Suite 104; phone No. 970– 625–5200).

Because some of the chemical analyses for your water well are still pending, I will be sending you another summary letter after receiving the remaining analytical results.

If you have any questions or would like to discuss these matters further, please contact me or Debbie Baldwin at the COGCC in Denver via e-mail (robert.chesson@state.co.us); Debbie.baldwin@state.co.us) or by phone at 303-894-2100, extension 112 (Bob) or 111 (Debbie).

Respectfully,

Robert H. Chesson, C.P.G., P.G.

Environmental Protection Specialist

Enclosures

cc:	Rich Griebling – COGCC w/o enclosures

Debbie Baldwin – COGCC w/o enclosures

Jamie Adkins – COGCC w/o enclosures

Brian Macke – COGCC w/o enclosures