October 22, 2004

VIA EMAIL

Mr. Chris R. Williams

EnCana Oil & Gas (USA) Inc.

950 17th Street, Suite 2600

Denver, Colorado 80202

RE:	Ground Water Remediation Proposal

West Divide Creek Seep Remediation No. 1815

SENW Section 12, Township 7 South; Range 92 West

Garfield County, Colorado

Dear Mr. Williams:

The Colorado Oil & Gas Conservation Commission (COGCC) staff has reviewed EnCana’s ground water remediation proposal for the West Divide Creek Area (dated September 8, 2004 and received by the COGCC on October 12, 2004) and the benzene isocontour map received on October 21, 2004.   At this time and based on the data presented, the COGCC staff is unable to approve the September 8, 2004 proposed plan.  The COGCC staff has not received the following previously requested items which are needed to conduct a meaningful technical review of the proposed remediation technique and to evaluate its appropriateness for mitigating the shallow ground water contamination at the West Divide Creek seep.  These items include:

- Contoured water table map(s) to identify ground water flow direction(s) and gradient(s) and a written evaluation of seasonal fluctuations of flow, flow direction(s), and gradient(s).

- A map(s) identifying losing-gaining stretches of West Divide Creek through the seep study area and a written discussion of your findings.

- A map(s) illustrating monthly variations of the following:  BTEX, dissolved methane, total dissolved solids, and presence of biogenic – thermogenic methane in any monitoring well or point with a dissolved methane concentration of 2 mg/l or greater and a written discussion of your findings.

- A map or series of maps that pulls together monitoring data from all of the various contractors that have been involved with your investigation and remediation and a written synthesis of their findings.  As an example, your isobenzene map shows the locations of the monitoring wells and the Colorado Mountain College (CMC) monitoring locations, but only shows the benzene concentrations at the monitoring wells.  CMC has collected samples from discharging ground water at a number of locations along West Divide Creek.  Benzene has been detected in some of these samples.  These data need to be included in any evaluation of the aerial extent of the benzene plume.

- Initially Cordilleran conducted two soil gas surveys the results of these surveys need to be incorporated into your remediation plan and evaluated to determine whether they indicate the occurrence of ground water impacts in areas where monitoring wells have not been or can not be installed.  These soil survey maps should be incorporated into whatever mapping system you have used to create the isobenzene map.

- It is our understanding that the survey of the 9 square mile area around the seep area and Schwartz well is underway.  If that work has added any other information about the aerial extent of impacts to the shallow ground water in West Divide Creek, then it needs to be included in your proposed work plan

Please note that delivery of raw monitoring data to the COGCC does not constitute a proper evaluation on EnCana’s part of what all of the data are indicating.  A large amount of data have been collected by your contractors, but it does not appear that anyone has taken the time to synthesize all of this information into a cohesive picture of site conditions from which rational decisions about proper remediation and long term monitoring can be made.  The COGCC staff does not have the time to create maps and piece together data that EnCana should have prepared in support of a COGCC order required submittal. Also note that lack of thoroughness by EnCana to COGCC requests on order required submittals may result in additional enforcement actions.

Proposed Remediation Technology

The COGCC staff is familiar with the conventional air sparging/soil vapor recovery technology; however we are not familiar with the proposed air sparging technology.  We require presentation of a case study or studies demonstrating that this proposed technique can create a ground water circulation cell(s) in the subsurface alluvial material present at the seep site and provide an appropriate remedial option to address the shallow ground water contamination along West Divide Creek.

Proposed Pilot Remediation Well

Although the COGCC staff cannot approve EnCana’s overall proposed remediation plan at this time, we have no objection to EnCana proceeding with a demonstration/pilot well at the seep site.  The information regarding the pilot test presented in the remediation plan; however, is very cursory and the COGCC requests additional information regarding how the pilot test will be conducted.

General Discussion

The COGCC staff has several concerns regarding the proposed location of the “remediation system”.  It appears that the ground water impacts are being viewed as if they were caused by a spill at some point source, and that the contamination is moving along with the ground water.  The proposed remediation appears to use a ground water plume capture system at the toe of the “plume”, but this ignores the significant hydrogeological issue that the conduits for the ground water contamination are likely intersecting fracture systems.  This was readily observable in the soil gas surveys conducted at the seep and discussed by Dr. Gorody during the hearing.  Additionally, no evidence has been presented that the shallow ground water contamination “plume” is actually migrating in the down gradient direction (or what is assumed to the down gradient direction due to the lack of contoured water data with which to evaluate ground water and contaminate flow direction(s)).  It is the staff opinion that an alternative configuration of remediation wells would be necessary to address the mechanism of release to the shallow ground water at the seep site.  This configuration would consist of remediation wells in the north-south direction (along the main “fracture” trend) in addition to some configuration of cross gradient wells.   Without some version of “plume” axis remediation, any remediation system would likely exist only as “plume” containment and not truly address the actual remediation of the shallow ground water impact.

The COGCC would like to discuss these topics with EnCana and their consultants the week of October 24, 2004 at our office if this allows for sufficient time to prepare the requested data presentations.  Please contact me or Debbie Baldwin (303) 894-2100 (ext. 112 Bob or ext. 111 Debbie) to arrange a date and time.

Respectfully,

Robert H. Chesson, C.P.G., P.G.

Environmental Protection Specialist

cc:	Debbie Baldwin – COGCC

Jamie Adkins – COGCC

Brian Macke – COGCC

Morris Bell – COGCC

Diane Bliezner, Esq. – EnCana

Eric Marsh, EnCana

Joel Fox, EnCana

Dave Petrie, EnCana

Doug Dennison – GARCO