July 2, 2004

Mr. Gary Hill

Hill Ranch

2473 County Road 336

Silt, Colorado 81652

RE:	COGCC Complaint No. 200056391

Stock Ponds in Section 9, Township 7 South, Range 92 West

Garfield County, Colorado

Dear Mr. Hill:

As discussed in our phone conversation on July 2, 1004, attached please find a copy of the laboratory analytical data results for the surface water sampling of your two stock watering ponds which was conducted by the Colorado Oil & Gas Conservation Commission (COGCC) on June 23, 2004.

The sampling of your two stock ponds was in response to a citizen’s concern regarding possible gas bubbles in the ponds observed prior on or before June 28, 2004.  Lisa Bracken notified the COGCC (Jaime Adkins, The Northwest Colorado Area Petroleum Engineer) of her concerns regarding bubbles in your ponds.  The subsequent pond sampling was conducted for the COGCC by Cordilleran Compliance Services (CCS), Inc. of Grand Junction, Colorado.  CCS sampled water from both stock ponds.  The water samples were submitted to Evergreen Analytical Laboratory (Evergreen) in Wheat Ridge, Colorado, for analysis of inorganic chemical constituents, organic compounds associated with petroleum hydrocarbons, methane gas, and pH.  The chemical constituents are evaluated to provide an overview of the pond’s general water quality.  Additionally, CCS collected samples of the gas bubbling in both ponds for the determination of isotopic and gas composition which can be used to determine the origin of the gas. The gas samples were submitted to Isotech Laboratories (Isotech) in Champaign, Illinois.  Copies of the laboratory analytical reports are enclosed.  The sample identification is the date of sample collection (062304) plus NP for north pond (north of County Road 336) and SP for south pond (South of County Road 336).

The Water Quality Control Commission (WQCC) of the Colorado Department of Public Health and Environment (CDPHE) has established drinking water standards for the protection of human health. The analytical results from the water samples from your ponds have been compared to applicable ground water and/or drinking water standards and are summarized below.  Please keep in mind that these water standards have been established for public drinking water supplies and not stock watering ponds.

### COMPARISON OF INORGANIC ANALYTICAL RESULTS TO STANDARDS

(Please see enclosed Evergreen Analytical Laboratory Report)

- Total Dissolved Solids (TDS):  CDPHE has established a TDS standard for human drinking water of 500 milligrams per liter (mg/l).  Although CDPHE does not have an agricultural standard for TDS, other agencies recommend concentrations below 2,000 mg/l for irrigation, and below 5,000 mg/l for most livestock watering.  TDS concentrations are related to the presence of naturally occurring elements and chemical compounds such as chloride, sodium, potassium, calcium, magnesium, and sulfate.

##### TDS was detected in the water sample from your South Pond at a concentration of 1,890 mg/l and a concentration of 160 mg/l for your North Pond.  The TDS concentration in your South Pond is above the CDPHE human drinking water standard, below the recommended maximum concentration for irrigation, and below the recommended maximum concentration for most livestock watering. The concentration in your North Pond is within the CDPHE human drinking water standard.

- Sodium (Na):  Although CDPHE does not have a standard for sodium, people on salt restricted diets should be aware of the Na concentration in the water they drink.  A concentration of drinking water with a concentration of sodium less than 20 mg/l is recommended by some for people on salt restricted diets or for people suffering from hypertension or heart disease.   Sodium occurs naturally in the ground water in many areas at concentrations that exceed the recommended level.

Sodium was detected in the water sample from your South Pond at a concentration of 320 mg/l, which is above the recommended level for people on salt restricted diets. The concentration of sodium in the water sample from your North Pond is 7.3 mg/l, which is within recommended levels for human consumption.

- Fluoride (F):  CDPHE has established a fluoride (F) standard for human drinking water is 4.0 mg/l.  Where fluoride concentrations are in the range of 0.7 mg/l to 1.2 mg/l, health benefits such as reduced dental decay have been observed.  Consumption of fluoride at concentrations of greater than 2.0 mg/l can result in mottling of teeth.  Consumption of fluoride at concentrations greater than 4.0 mg/l can increase the risk of skeletal fluorosis or other adverse health effects. 

Fluoride was detected in the water sample from your South Pond at a concentration 1.6 mg/l, which is below the human health drinking standard and within the concentration range where beneficial health effects are observed.  Fluoride was not detected in the water sample from your North Pond.

- Chloride (Cl):  The CDPHE chloride standard for drinking water is 250 mg/l.   Chloride concentrations in excess of 250 mg/l usually produce a noticeable taste in drinking water.

Chloride was detected in the water sample from your South Pond at a concentration of 177 mg/l, which is below the human drinking water standard.  Chloride was detected in the water sample from your North Pond at a concentration of 1.6 mg/l, also within the human drinking water standard.

- Sulfate (SO4):  The CDPHE sulfate standard for drinking water is 250 mg/l.  Although CDPHE does not have an agricultural standard for sulfate, other agencies recommend a concentration below 1,500 mg/l for livestock watering.  Waters containing high concentrations of sulfate, typically caused by the leaching of natural deposits of magnesium sulfate (Epsom salts) or sodium sulfate (Glauber's salt), may be undesirable because of their laxative effects. Sulfate occurs naturally in the ground and surface water in many areas in Colorado at concentrations that exceed the drinking water standard.

Sulfate was detected in the water sample from your South Pond at a concentration of 5.82 mg/l, which is below the CDPHE drinking water standard and below the recommended maximum concentration for livestock watering.  The sulfate concentration in the sample from your North Pond is 3.1 mg/l.

- Total Nitrate (NO3) + Nitrite (NO2) as Nitrogen (N):  The CDPHE total nitrate (NO3) + nitrite (NO2) as nitrogen (N) for standard for human drinking water is 10 mg/l.  Nitrate and nitrite are common contaminants in ground water from agricultural sources, such as fertilizer and animal, including human, wastes.  They are known to cause infant cyanosis or “blue baby disease” in humans and, at concentrations greater than 100 mg/l as nitrogen (N), may be dangerous to livestock.  High concentrations of nitrate and nitrite in ground water are known to occur in agricultural areas in Colorado.

Total nitrate/nitrite as N was not detected in either the South Pond or North Pond samples.

- Iron (Fe):  The CDPHE standard for human drinking water for iron is 0.3 mg/l.  Small amounts of iron are common in ground water and surface water.  Iron may produce a brownish-red color in laundered clothing, can leave reddish stains on fixtures, and impart a metallic taste to beverages and food made with it.  After a period of time iron deposits can build up in pressure tanks, water heaters, and pipelines, reducing the effective flow rate and efficiency of the water supply.

Iron was detected at a concentration of 20 mg/l in the water sample from your South Pond and at a concentration of 1.5 mg/l in the water sample from your North Pond.  Both sample concentration results are above the CDPHE drinking water standard.

- Selenium (Se): The CDPHE selenium standard for human drinking water is 0.05 mg/l and the agricultural standard is 0.02 mg/l.  Excessive selenium (Se) (concentrations greater than 0.05 mg/l) can cause loss of hair and/or fingernails as well as adverse effects on the central nervous system.  Selenium (Se) occurs naturally in the ground and surface water in many areas of Colorado at concentrations that exceed the drinking water standard.

Selenium was detected in the water sample from your South Pond at a concentration of 0.014 mg/l, which is within the CDPHE human drinking water standard.  Selenium was not detected in the water sample from your North Pond.

Several other metals, such as arsenic, barium, copper, and lead were also detected in the water samples.  The following table presents a summary of these data.  Some of these metals do not have drinking water standards. If one of the samples from your ponds exceeds the concentration standard I have presented the text in bold.

Table 1

| Metal | South Pond | North Pond | Drinking Water | Secondary Drinking Water Standard or Agricultural Standard |
| --- | --- | --- | --- | --- |
| Arsenic (As) | 0.089 | 0.042 | 0.05 | 0.1 |
| Barium (Ba) | 0.92 | 0.16 | 2.0 | No standard |
| Calcium (Ca) | 110 | 33 | No Standard | No Standard |
| Chromium (Cr) | 0.023 | Not Detected | 0.1 | 0.1 |
| Copper (Cu) | 0.038 | Not Detected | No Standard | 0.1 |
| Lead (Pb) | 0.025 | Not Detected | 0.05 | 0.1 |
| Magnesium (Mg) | 54 | 6.1 | No Standard | No Standard |
| Manganese (Mn) | 1.2 | 0.19 | No Standard | 0.2 |
| Potassium (K) | 81 | 4.6 | No Standard | No Standard |

All of the concentrations presented on Table 1 are in mg/l.

ORGANIC COMPOUNDS ASSOCIATED WITH PETROLEUM HYDROCARBONS

(please see enclosed Evergreen Analytical METHOD E 602/SW 8021B, BTEX DATA REPORT)

- Benzene: CDPHE's basic ground water standard for benzene is 5 micrograms per liter (µg/l).  Benzene was not detected in either of your two stock ponds.

- Toluene: CDPHE's basic ground water standard for toluene is 1,000 µg/l.  Toluene was not detected in either of your two stock ponds.

- Ethylbenzene: CDPHE's basic ground water standard for ethylbenzene is 680 µg/l.  Ethylbenzene was not detected in either of your two stock ponds.

- Total Xylenes (sum of m,p, and o-xylene): CDPHE's basic ground water standard for total xylenes is 10,000 µg/l.  Total xylenes were not detected in either of your two stock ponds.

METHANE GAS CONCENTRATION

(Please see enclosed Evergreen Analytical Laboratory METHANE REPORT)

- Methane was detected in the sample from your North Pond at a concentration of 0.056 mg/l.  Methane was not detected in the sample from your South Pond.

GAS COMPOSITIONAL AND ISOTOPIC ANALYTICAL RESULTS

(See attached Isotech Laboratories, Inc. report)

The ratios of the stable isotopes and detailed compositional analysis of methane gas can be used to differentiate the origin and sources of methane.  The results of the isotopic and compositional analysis are summarized on the Isotech Laboratories report.

Isotopic Analysis of Methane (North Pond):

- The deuterium/hydrogen isotope ratio for the methane in the water sample from your North Pond is –387.3

- The carbon-13/carbon-12 isotope ratio for the methane in the water sample from your well is -55.14

Isotopic Analysis of Methane (South Pond):

- The deuterium/hydrogen isotope ratio for the methane in the water sample from your North Pond is –357.6

- The carbon-13/carbon-12 isotope ratio for the methane in the water sample from your well is -55.14

The gas in both of your stock ponds is biogenic methane, which is formed by bacteriological activity (such as swamp gas, sewer gas, etc.).  Specifically gas researchers have identified as the type of biogenic methane found in your ponds as Near Surface Microbial Gas as the result of biological fermentation.   I have included a cross-plot of the stable methane isotopes for your stock ponds.  You will notice the area near the top right corner as defined a “Thermogenic Gas”.  This is the area of the cross-plot that the natural gas produced by the gas wells in the Mamm Creek Gas Field plot.  Thermogenic means methane gas produced by geologic processes.  The gas compositional analysis indicates that the gas samples collected from your ponds area made up of a mixture of air (nitrogen, oxygen, etc.) and methane. No other amounts of “heavier” hydrocarbon gases (such as propane or butane) are present. This is also very common in gas produced by biological activity.  If the methane gas in your ponds was due to a release from a gas production well, not only would the gas cross-plot as thermogenic, the gas composition would contain those “heavier” hydrocarbon gases.

CONCLUSION

Methane gas stable isotope and gas composition results for both of your stock ponds (North Pond and South Pond) shows that the bubbling observed is due to natural biogenic methane gas produced by bacteria.  Additionally, overall water quality of your ponds show chemical conditions of both ponds within conditions that would be expected for surface water stock ponds.

Should you have any questions, please call me at (303) 894-2100 X112.

Respectfully,

Robert H. Chesson

Environmental Protection Specialist

cc:  Debbie Baldwin - COGCC