May 25, 2004

VIA FACIMILIE

Mr. Pepi Langegger

3807 Lupine Drive

Vail, Colorado 81657

Ms. Lisa Bracken

P.O. Box 30

Silt, Colorado 81652

RE:	Water Quality Analytical Results

Section 12 – Township 7 South – Range 92 West

Garfield County, Colorado

West Divide Creek Gas Seep Investigation

COGCC Investigation No. 1815

Dear Mr. Langegger and Ms. Bracken:

On May 19, 2004, I collected a water sample from a water seep on the eastern bank of West Divide Creek adjacent to the main gas seep area.  Ms. Bracken had requested that the COGCC sample a suspicious oil sheen like occurrence related to the water seeps although it is the COGCC’s staff opinion that the oil sheen material is a naturally occurring bacterial biofilm.  The water sample was collected from a small seep feature which was discharging a very low volume of water in addition to bubbling natural gas as a result of the gas seep.  Included in the sample was an extensive amount of the sheen-like biofilm.  The sample was submitted to Evergreen Analytical Laboratory (Evergreen) in Wheat Ridge, Colorado, for analysis of benzene, toluene, ethylbenzene, and total xylenes (BTEX).  The results are attached along with a location map of the sample location.    Benzene was detected, along with low concentrations of toluene, ethylbenzene, and xylenes.  These compounds are the result of the EnCana gas release and indicate an impact to shallow groundwater adjacent to the main seep area.  The COGCC is in discussions with both EnCana and their consultant regarding the investigation of this ground water impact.  It is likely that EnCana or their representatives will be contacting both of you regarding onsite actions regarding this part of the West Divide Creek gas seep remediation.

If you have any questions or would like to discuss these matters further, please contact me or Debbie Baldwin at the COGCC in Denver via e-mail (robert.chesson@state.co.us); debbie.baldwin@state.co.us) or by phone at 303-894-2100, extension 112 (Bob) or 111 (Debbie).

Respectfully,

Robert H. Chesson, C.P.G., P.G.

Environmental Protection Specialist

Enclosures

cc:	Rich Griebling – COGCC w/o enclosures

Debbie Baldwin – COGCC w/o enclosures

Jamie Adkins – COGCC w/o enclosures

Brian Macke – COGCC w/o enclosures