West Divide Creek Natural Gas Seep

Where is the natural gas seep on West Divide Creek?

The seep is located in West Divide Creek drainage (Section 12, Township 7 South, Range 92 West), approximately six miles south of Silt, Colorado.  The area of the creek initially affected by the seep was approximately 200 yards in extent.  Because of …… the current size and vigor of the gas seep has diminished ………

What is the cause of the natural gas seep in West Divide Creek?

The natural gas seep was caused by a subsurface release of gas from a natural gas production well, the Schwartz 2-15B operated by EnCana Oil & Gas (EnCana). The Schwartz 2-15B is located Section 2, Township 7 South, Range 92 West approximately 3,000 feet northwest of the seep.  This natural gas production well was completed in early …(need date)…….  into the Williams Fork Formation at a depth of approximately 6,400 feet below the ground surface.

Did loss of well control during a fracture stimulation (“fracing”) cause the release of the natural gas from the Schwartz 2-15B well?

No. The release of the natural gas from the Schwartz 2-15B well was the result of a failure of the cement used to isolate the well production casing.  This failure resulted in the loss of approximately 4,000 feet of cement from the top of the well bore allowing a portion of the well bore to remain “open”.  This “open” section of the well bore allowed for natural gas from deeper in the well to migrate through the shallow subsurface rock strata and into natural faults and fractures and into West Divide Creek.  At no time during the well completion are there any records of pressure changes suggesting any “loss of control” during or after the fracing. Additionally, EnCana used carbon dioxide gas in addition to water for the fracing at the Schwarts 2-15B well.  Carbon dioxide gas does not occur with the natural gas in the Williams Fork Formation.  Only very low concentrations of carbon dioxide gas have been detected in the natural gas samples collected from the West Divide Creek gas seep.  That carbon dioxide gas is the result of mixing of the seep gas with the atmosphere.

How does the COGCC know that the Schwarts 2-15B well was the cause of the gas seep and not another production well in the area?

The COGCC staff has investigated other production wells near the West Divide Creek gas seep as possible causes of the seep. Remedial cementing to replace the 4,000 feet of lost cement on the Schwarts 2-15B well, was conducted on April 5, 2004.  Since that date the volume and size of the gas seep on West Divide Creek has decreased significantly.

What about the presence on benzene in the creek water at the West Divide Creek natural gas seep?

Sampling of the creek water directly at the main seep has had low concentrations of benzene.  The initial water sample collected on April 2, 2004 had a benzene concentration of 99 micrograms per liter (g/l).  Subsequent sampling has shown the concentration of benzene in the creek water to have decreased to

shown the presence of benzene, toluene, and m,p xylenes.  The initial water sample was collected on April 2, 2004 and laboratory analysis yielded and 99, 100, and 17 micrograms per liter), respectively

What type of operational changes has the COGCC enacted for Oil and Gas drilling operations in the West Divide Creek area that will assure that a release like the natural gas release from the Schwartz well will not happen again?

When will the COGCC allow EnCana to resume well drill and completions in the Divide Creek area?