April 28, 2004

Mr. Pepi Langegger

3807 Lupine Drive

Vail, Colorado 81657

RE:	Water Quality Analytical Results for Stream and Pond Samples

Section 12 – Township 7 South – Range 92 West

Garfield County, Colorado

West Divide Creek Gas Seep Investigation

COGCC Investigation No. 1802

Dear Mr. Langegger:

As you are aware, on April 8, 2004, Debbie Baldwin of the Colorado Oil and Gas Conservation Commission (COGCC) sampled both West Divide Creek and two stock ponds adjacent to the creek for evidence of possible impact from a natural gas production well release.  This release is the likely cause of the gas seep recently developed in West Divide Creek.   The COGCC has received the results for the analyses conducted on these water samples and methane gas composition and stable isotope samples and this letter presents these results.  The water well samples were submitted to Evergreen Analytical Laboratory (Evergreen) in Wheat Ridge, Colorado, for analysis of inorganic chemical constituents, organic compounds associated with petroleum hydrocarbons, and methane gas.  The gas composition and stable isotope samples were submitted to Isotech Laboratories (Isotech) in Champaign, Illinois.  Copies of the laboratory analytical reports are enclosed.

The sample locations are identified by the following names:

- West Divide Creek – Upstream (of the gas seep)

- Langegger Upper Stock Pond

- Langegger Lower Stock Pond

- Beaver Pond; West Divide Creek – Downstream (of the gas seep)

I have included a location map for these sample sites.

The Water Quality Control Commission (WQCC) of the Colorado Department of Public Health and Environment (CDPHE) has established drinking water standards for the protection of human health. The analytical results from these water samples have been compared to applicable ground water and/or drinking water standards and are summarized below.  Please keep in mind that these water standards were established for public drinking water supplies and not for aquatic life or livestock watering.  Often people use and consume ground water from private wells and springs, or water livestock with water that can exceed these standards.

### COMPARISON OF INORGANIC ANALYTICAL RESULTS TO STANDARDS

(Please see enclosed Evergreen Analytical Laboratory Report)

- Sodium (Na):  Although CDPHE does not have a standard for sodium, people on salt restricted diets should be aware of the Na concentration in the water they drink.  A concentration of drinking water with a concentration of sodium less than 20 mg/l is recommended by some for people on salt restricted diets or for people suffering from hypertension or heart disease.   Sodium occurs naturally in the ground water in many areas at concentrations that exceed the recommended level.
West Divide Creek – Upstream

Sodium was detected in the upstream water sample at a concentration of 29 “B” mg/l, which is slightly greater than the recommended level for people on salt restricted diets. The B notation is a laboratory “flag” which is used to indicate the presence of a laboratory-induced contaminant in your sample (known as blank [“B”] contamination).  In this case the laboratory noticed the presence of sodium in the analytical method control sample (called a method blank).  Normally the method blank does not have any compounds.  Because this analysis result is preliminary, the laboratory has not yet sent the laboratory analysis quality control summary that discusses the magnitude of the blank contamination.

Langegger Upper Stock Pond

Sodium was detected in the water sample from your upper stock pond at a concentration of 36 “B” mg/l, which is greater than the recommended level for people on salt restricted diets.

Langegger Lower Stock Pond

Sodium was detected in the water sample from your lower stock pond at a concentration of 55 “B” mg/l, which is greater than the recommended level for people on salt restricted diets.

Beaver Pond; West Divide Creek – Downstream

Sodium was detected in the water sample from your well at a concentration of 36 “B” mg/l, which is greater than the recommended level for people on salt restricted diets.

- Fluoride (F):  CDPHE has established a fluoride (F) standard for human drinking water is 4.0 mg/l.  Where fluoride concentrations are in the range of 0.7 mg/l to 1.2 mg/l, health benefits such as reduced dental decay have been observed.  Consumption of fluoride at concentrations of greater than 2.0 mg/l can result in mottling of teeth.  Consumption of fluoride at concentrations greater than 4.0 mg/l can increase the risk of skeletal fluorosis or other adverse health effects. 

West Divide Creek – Upstream

Fluoride was not detected in the upstream water sample.

Langegger Upper Stock Pond

Fluoride was detected in the water sample from your upper stock pond at a concentration of 0.84 mg/l, which is within the drinking water standard.

Langegger Lower Stock Pond

Fluoride was detected in the water sample from your lower stock pond at a concentration of 0.78 mg/l, which is within the drinking water standard.

Beaver Pond; West Divide Creek – Downstream

Fluoride was detected in the water sample from the beaver pond at a concentration of  0.44 mg/l, which is within the drinking water standard.

- Chloride (Cl):  The CDPHE chloride standard for drinking water is 250 mg/l.   Chloride concentrations in excess of 250 mg/l usually produce a noticeable taste in drinking water.

West Divide Creek – Upstream

Chloride was detected in the upstream water sample at a concentration of 4.17 mg/l, which is within the drinking water standard.

Langegger Upper Stock Pond

Chloride was detected in the water sample from your upper stock pond at a concentration of 3.49 mg/l, which is within the drinking water standard.

Langegger Lower Stock Pond

Chloride was detected in the water sample from your lower stock pond at a concentration of 3.02 mg/l, which is within the drinking water standard.

Beaver Pond; West Divide Creek – Downstream

Chloride was detected in the water sample from the beaver pond at a concentration of  4.58 mg/l, which is within the drinking water standard.

- Sulfate (SO4):  The CDPHE sulfate standard for drinking water is 250 mg/l.  Although CDPHE does not have an agricultural standard for sulfate, other agencies recommend a concentration below 1,500 mg/l for livestock watering.  Waters containing high concentrations of sulfate, typically caused by the leaching of natural deposits of magnesium sulfate (Epsom salts) or sodium sulfate (Glauber's salt), may be undesirable because of their laxative effects. Sulfate occurs naturally in the ground water in many areas in Colorado at concentrations that exceed the drinking water standard.

West Divide Creek – Upstream

Sulfate was detected in the upstream water sample at a concentration of 21.7 mg/l, which is within the drinking water standard and below the recommended maximum concentration for most livestock watering.

Langegger Upper Stock Pond

Sulfate was detected in the water sample from your upper stock pond at a concentration of 17.9 mg/l, which is within the drinking water standard and below the recommended maximum concentration for most livestock watering.

Langegger Lower Stock Pond

Sulfate was detected in the water sample from your lower stock pond at a concentration of 17.5 mg/l, which is within the drinking water standard and below the recommended maximum concentration for most livestock watering.

Beaver Pond; West Divide Creek – Downstream

Sulfate was detected in the water sample from the beaver pond at a concentration of  21.2 mg/l, which is within the drinking water standard and below the recommended maximum concentration for most livestock watering.

- Total Nitrate (NO3) + Nitrite (NO2) as Nitrogen (N):  The CDPHE total nitrate (NO3) + nitrite (NO2) as nitrogen (N) for standard for human drinking water is 10 mg/l.  Nitrate and nitrite are common contaminants in ground water from agricultural sources, such as fertilizer and animal, including human, wastes.  They are known to cause infant cyanosis or “blue baby disease” in humans and, at concentrations greater than 100 mg/l as nitrogen (N), may be dangerous to livestock.  High concentrations of nitrate and nitrite in ground water are known to occur in agricultural areas in Colorado.  

West Divide Creek – Upstream

Total nitrate/nitrite, as N was detected in the upstream water sample at a concentration of 0.0565 mg/l, which is within the drinking water standard

Langegger Upper Stock Pond

Total nitrate/nitrite, as N was not detected in the water sample from your upper stock pond.

Langegger Lower Stock Pond

Total nitrate/nitrite, as N was not detected in the water sample from your lower stock pond.

Beaver Pond; West Divide Creek – Downstream

Total nitrate/nitrite, as N was not detected in the water sample from the beaver pond.

- Iron (Fe):  The CDPHE standard for human drinking water for iron is 0.3 mg/l.  Small amounts of iron are common in ground water.  Iron may produce a brownish-red color in laundered clothing, can leave reddish stains on fixtures, and impart a metallic taste to beverages and food made with it.  After a period of time iron deposits can build up in pressure tanks, water heaters, and pipelines, reducing the effective flow rate and efficiency of the water supply.

West Divide Creek – Upstream

Iron was not detected in the upstream water sample.

Langegger Upper Stock Pond

Iron was not detected in the water sample from your upper stock pond.

Langegger Lower Stock Pond

Iron was not detected in the water sample from your lower stock pond.

Beaver Pond; West Divide Creek – Downstream

Iron was not detected in the water sample from the beaver pond.

- Selenium (Se): The CDPHE selenium standard for human drinking water is 0.05 mg/l and the agricultural standard is 0.02 mg/l.  Excessive selenium (Se) (concentrations greater than 0.05 mg/l) can cause loss of hair and/or fingernails as well as adverse effects on the central nervous system.  Selenium (Se) occurs naturally in the ground water in many areas of Colorado at concentrations that exceed the drinking water standard.

West Divide Creek – Upstream

Selenium was not detected in the upstream water sample.

Langegger Upper Stock Pond

Selenium was not detected in the water sample from your upper stock pond.

Langegger Lower Stock Pond

Selenium was not detected in the water sample from your lower stock pond.

Beaver Pond; West Divide Creek – Downstream

Selenium was not detected in the water sample from the beaver pond.

- Calcium (Ca), Potassium (K), Magnesium (Mg), Bicarbonate (HCO3) and Carbonate (CO3) were also tested for in your water.  There are no standards from CDPHE for these parameters.

ORGANIC COMPOUNDS ASSOCIATED WITH PETROLEUM HYDROCARBONS

(please see enclosed Evergreen Analytical METHOD E 602/SW 8021B, BTEX DATA REPORT)

- Benzene: CDPHE's basic ground water standard for benzene is 5 micrograms per liter (µg/l). Because West Divide Creek is classified by WQCC as a Water Supply Segment for the Lower Colorado River Basin, the CDPHE has a different benzene standard for surface water.   This benzene standard is 1.2 μg/l.

West Divide Creek – Upstream

Benzene was not detected in the upstream water sample.

Langegger Upper Stock Pond

Benzene was not detected in the water sample from your upper stock pond.

Langegger Lower Stock Pond

Benzene was not detected in the water sample from your lower stock pond.

Beaver Pond; West Divide Creek – Downstream

Benzene was not detected in the water sample from the beaver pond.

As you are aware, benzene has been detected in West Divide Creek in conjuncture with the active gas seep.  A COGCC surface water sample from the main seep collected on 4/2/04 had a benzene concentration of  99 μg/l. Subsequent water samples collected by EnCana in the same section of the creek has been showing a rapid decrease in benzene concentrations to around 10 μg/l or less.

- Toluene: CDPHE's basic ground water standard for toluene is 1,000 µg/l.

West Divide Creek – Upstream

Toluene was not detected in the upstream water sample.

Langegger Upper Stock Pond

Toluene was not detected in the water sample from your upper stock pond.

Langegger Lower Stock Pond

Toluene was not detected in the water sample from your lower stock pond.

Beaver Pond; West Divide Creek – Downstream

Toluene was not detected in the water sample from the beaver pond.

As you are aware, toluene has been detected in West Divide Creek in conjuncture with the active gas seep.  A COGCC surface water sample from the main seep collected on 4/2/04 had a toluene concentration of 100 μg/l. Subsequent water samples collected by EnCana in the same section of the creek has been showing a rapid decrease in toluene concentrations to around 10 μg/l or less.

- Ethylbenzene: CDPHE's basic ground water standard for ethylbenzene is 680 µg/l.

West Divide Creek – Upstream

Ethylbenzene was not detected in the upstream water sample.

Langegger Upper Stock Pond

Ethylbenzene was not detected in the water sample from your upper stock pond.

Langegger Lower Stock Pond

Ethylbenzene was not detected in the water sample from your lower stock pond.

Beaver Pond; West Divide Creek – Downstream

Ethylbenzene was not detected in the water sample from the beaver pond.

Ethylbenze has not been detected in West Divide Creek in conjuncture with the active gas seep.

- Total Xylenes (sum of m,p, and o-xylene): CDPHE's basic ground water standard for total xylenes is 10,000 µg/l.

West Divide Creek – Upstream

Total xylenes were not detected in the upstream water sample.

Langegger Upper Stock Pond

Total xylenes were not detected in the water sample from your upper stock pond.

Langegger Lower Stock Pond

Total xylenes were not detected in the water sample from your lower stock pond.

Beaver Pond; West Divide Creek – Downstream

Total xylenes were not detected in the water sample from the beaver pond.

As you are aware, total xylenes have been detected in West Divide Creek in conjuncture with the active gas seep.  A COGCC surface water sample from the main seep collected on 4/2/04 had a total xylenes concentration of 17 μg/l. Subsequent water samples collected by EnCana in the same section of the creek have not shown detectable concentrations of total xylenes.

METHANE GAS CONCENTRATION

(Please see enclosed Evergreen Analytical Laboratory METHANE REPORT)

West Divide Creek – Upstream

Dissolved methane gas was detected in the upstream water sample at a concentration of 0.0087 mg/l.

Langegger Upper Stock Pond

Dissolved methane gas was detected in the water sample from your upper stock pond at a concentration of 0.10 mg/l.

Langegger Lower Stock Pond

Dissolved methane gas was detected in the water sample from your lower stock pond at a concentration of 0.76 mg/l.

Beaver Pond; West Divide Creek – Downstream

Dissolved methane gas was detected in the water sample from the beaver pond at a concentration of 0.025 mg/l.

Methane gas alone is physiologically inert and non-toxic to humans.  Normal breath exhalation contains 1 to 99 ppm of methane (parts per million [ppm] is the same units as mg/l).  The presence of methane in drinking water does not present a known health hazard to humans or other animals via ingestion; however, methane in domestic water supplies can be associated with undesirable and potentially serious side effects.

Methane gas dissolved in water “exsolves” when exposed to the atmosphere and dissipates rapidly because it is lighter than air.  This is often responsible for the “fizzing” observed in water wells that may contain methane gas.  If the methane occurs at a high enough concentration and if it is allowed to accumulate in a confined space, such as a well pit, crawl space, closet, etc., an explosion hazard can be established.  In addition, if methane concentrations in well water are high, then pockets of free gas form within the water and cause the well pump to cavitate and no longer bring water to the surface.

Methane gas is common in water wells and surface water impoundments in Colorado.  It occurs naturally and the source of the methane is commonly from one or more of the sources listed below.

- Methane is commonly found as a gas in coal or black (organic rich) shale seams in the subsurface.

- Methane is a byproduct of the decay of organic matter.

GAS COMPOSITIONAL AND ISOTOPIC ANALYTICAL RESULTS

(see attached Isotech Laboratories, Inc. report)

Gas samples for determination of gas composition and stable isotopes of methane were collected from two surface sample locations on your property; the Upper Stock Pond and the Lower Stock Pond.  No sample was collected at the Upper stream location on West Divide Creek and due to lack of obvious gas bubbling; no sample was collected from the Beaver Pond.  The ratios of the stable isotopes and detailed compositional analysis of the gases can be used to differentiate the origin and sources of methane.  The results of the isotopic and compositional analysis are summarized on the Isotech Laboratories report.

Langegger Upper Stock Pond

Isotopic Analysis of Methane:

The deuterium/hydrogen isotope ratio for the methane in the water sample from your Upper Pond is –361.5.

The carbon-13/carbon-12 isotope ratio for the methane in the water sample from your Upper Pond is -64.65

The gas in your upper stock pond is biogenic methane, which is formed by bacteriological activity (such as swamp gas, sewer gas, etc.).  The gas compositional analysis indicates that the gas sample is made up of a mixture of air (nitrogen, oxygen, argon, and carbon dioxide), methane with very small (trace) amount of other hexanes.  Previous observations of gas seep activity by COGCC staff (April 1, 2004 and April 2, 2004) did indicate that your Upper Stock Pond was likely impacted by the gas seep; however,  this impact has decreased rapidly so that by April 8 , 2004 no evidence of gas impact from the seep in your Upper Pond was evident, either visually or through analytical sampling.

Langegger Lower Stock Pond

Isotopic Analysis of Methane:

The deuterium/hydrogen isotope ratio for the methane in the water sample from your Lower Pond is –207.1.

The carbon-13/carbon-12 isotope ratio for the methane in the water sample from your well is -43.05

The gas in your Lower Stock Pond is thermogenic methane, which is methane gas from geologic processes as opposed to biogenic methane.  The gas compositional analysis indicates that the gas sample is made up of a mixture of air (nitrogen, oxygen, etc.) and methane, ethane, and propane with very small (trace) amounts of other hydrocarbon gases.  It is the COGCC staff’s opinion that the methane gas in your Lower Pond is the same as the gas from the seep in West Divide Creek, which is identical to the gas produced from the Williams Fork Formation in the natural gas production wells in the area.

CONCLUSION

With the exception of sodium (Na) no other organic or inorganic compounds detected in the April 8, 2004 sampling of the four sample sites on your property exceeded any of the CDPHE drinking water health advisories or standards.  Although the concentrations of sodium detected are relatively low, because you or your livestock and pets may drink water form the creek, you may wish to discuss the possible health effects of continued consumption with your physician and/or veterinarian.

Other than the presence of the methane gas in your Lower Stock Pond, none of the other three sample locations (West Divide Creek – Upstream, Upper Stock Pond, and the Beaver Pond) showed any impact from the gas seep on West Divide Creek.  EnCana is continuing with periodic sampling of the active seep areas within West Divide Creek and has implemented both active (air sparging) and passive (activated charcoal booms) remediation actions along the impacted section of West Divide Creek.  As of this date, detectable concentrations of BTEX compounds in the creek have decreased and the rate and overall volume of seeping gas at the active seep appears to be decreasing.  Further investigation of the gas release and seep impact is continuing by both EnCana and the COGCC staff.   The COGCC will release any new information on this investigation as soon as it is available.

If you have any questions or would like to discuss these matters further, please contact me or Debbie Baldwin at the COGCC in Denver via e-mail (robert.chesson@state.co.us); Debbie.baldwin@state.co.us) or by phone at 303-894-2100, extension 112 (Bob) or 111 (Debbie).

Respectfully,

Robert H. Chesson, C.P.G., P.G.

Environmental Protection Specialist

Enclosures

cc:	Rich Griebling – COGCC w/o enclosures

Debbie Baldwin – COGCC w/o enclosures

Jamie Adkins – COGCC w/o enclosures

Brian Macke – COGCC w/o enclosures