July 27, 2004

Mr. Gary Gagne

Ms. Nancy Jacobsen

P.O. Box 1238

Glenwood Springs, Colorado 81602

RE:	Water Quality Analytical Results for Your Water Well

Section 10 – Township 7 South – Range 92 West

Garfield County, Colorado

West Divide Creek Gas Seep Investigation

COGCC Remediation No. 1815

Dear Mr. Gagne and Ms. Jacobsen:

On June 30, 2004, Debbie Baldwin and Bob Chesson of the Colorado Oil and Gas Conservation Commission (COGCC) sampled your water well as part of the continuing investigation into possible impact from a natural gas production well release near your residence.   The water samples from your well were submitted to Evergreen Analytical Laboratory (Evergreen) in Wheat Ridge, Colorado, for analysis of inorganic chemical constituents, organic compounds associated with petroleum hydrocarbons, and methane gas.  In addition, a sample of gas from your water well was collected for determination of gas composition and stable isotopes of methane.  This sample was submitted to Isotech Laboratories (Isotech) in Champaign, Illinois.  Copies of the laboratory analytical reports are enclosed.  The COGCC previously sampled your water well on April 9, 2004, as part the West Divide Creek gas seep investigation and I have compared the recent analytical results (June 30, 2004) with the April 2004 results.

The Water Quality Control Commission (WQCC) of the Colorado Department of Public Health and Environment (CDPHE) has established drinking water standards for the protection of human health. The analytical results from the water samples from your well have been compared to applicable ground water and/or drinking water standards and are summarized below.  Please keep in mind that these water standards were established for public drinking water supplies.  Often people use and consume ground water from private wells that can exceed these standards.

### COMPARISON OF INORGANIC ANALYTICAL RESULTS TO STANDARDS

(Please see enclosed Evergreen Analytical Laboratory Report)

- Total Dissolved Solids (TDS):  CDPHE has established a TDS standard for human drinking water of 500 milligrams per liter (mg/l).  Although CDPHE does not have an agricultural standard for TDS, other agencies recommend concentrations below 2,000 mg/l for irrigation, and below 5,000 mg/l for most livestock watering.  TDS concentrations are related to the presence of naturally occurring elements and chemical compounds such as chloride, sodium, potassium, calcium, magnesium, and sulfate.

##### TDS was detected in your water well at a concentration of 1,020 mg/l, which is greater than the CDPHE human drinking water standard, below the recommended maximum concentration for irrigation, and below the recommended maximum concentration for most livestock watering.  The sample results for the April 9, 2004 sample was 1,060 mg/l.

- Sodium (Na):  Although CDPHE does not have a standard for sodium, people on salt restricted diets should be aware of the Na concentration in the water they drink.  A concentration of drinking water with a concentration of sodium less than 20 mg/l is recommended by some for people on salt restricted diets or for people suffering from hypertension or heart disease.   Sodium occurs naturally in the ground water in many areas at concentrations that exceed the recommended level.

Sodium was detected in the water sample from your well at a concentration of 390 mg/l, which is greater than the recommended level for people on salt restricted diets. The sodium concentration for the April 9, 2004 sample was 380 mg/l.

- Fluoride (F):  CDPHE has established a fluoride (F) standard for human drinking water is 4.0 mg/l.  Where fluoride concentrations are in the range of 0.7 mg/l to 1.2 mg/l, health benefits such as reduced dental decay have been observed.  Consumption of fluoride at concentrations of greater than 2.0 mg/l can result in mottling of teeth.  Consumption of fluoride at concentrations greater than 4.0 mg/l can increase the risk of skeletal fluorosis or other adverse health effects. 

Fluoride was detected in the water sample from your well at a concentration of 4.8 mg/l, which is slightly above the human drinking water standard. The fluoride concentration for the April 9, 2004 sample was 4.5 mg/l.

- Chloride (Cl):  The CDPHE chloride standard for drinking water is 250 mg/l.   Chloride concentrations in excess of 250 mg/l usually produce a noticeable taste in drinking water.

Chloride was detected in the water sample from your well at a concentration of 254 mg/l, which is slightly above the CDPHE drinking water standard.  The chloride concentration for the April 9, 2004 sample was 209 mg/l.

Sulfate (SO4):  The CDPHE sulfate standard for drinking water is 250 mg/l.  Although CDPHE does not have an agricultural standard for sulfate, other agencies recommend a concentration below 1,500 mg/l for livestock watering.  Waters containing high concentrations of sulfate, typically caused by the leaching of natural deposits of magnesium sulfate (Epsom salts) or sodium sulfate (Glauber's salt), may be undesirable because of their laxative effects. Sulfate occurs naturally in the ground water in many areas in Colorado at concentrations that exceed the drinking water standard.

- Sulfate was detected in the water sample from your well at a concentration of 204  mg/l, which is below the CDPHE drinking water standard and below the recommended maximum concentration for most livestock watering. The sulfate concentration for the April 9, 2004 sample was 258 mg/l.

- Total Nitrate (NO3) + Nitrite (NO2) as Nitrogen (N):  The CDPHE total nitrate (NO3) + nitrite (NO2) as nitrogen (N) for standard for human drinking water is 10 mg/l.  Nitrate and nitrite are common contaminants in ground water from agricultural sources, such as fertilizer and animal, including human, wastes.  They are known to cause infant cyanosis or “blue baby disease” in humans and, at concentrations greater than 100 mg/l as nitrogen (N), may be dangerous to livestock.  High concentrations of nitrate and nitrite in ground water are known to occur in agricultural areas in Colorado.  

Total nitrate/nitrite, as N was detected in the sample from your well at a concentration of 11 mg/l, which is above the human health standard for drinking water. You should make sure that you remove any potential sources of nitrate/nitrite from the area around or upgradient of your water well. The total nitrate/nitrite concentration for the April 9, 2004 sample was 3.4 mg/l.

- Iron (Fe):  The CDPHE standard for human drinking water for iron is 0.3 mg/l.  Small amounts of iron are common in ground water.  Iron may produce a brownish-red color in laundered clothing, can leave reddish stains on fixtures, and impart a metallic taste to beverages and food made with it.  After a period of time iron deposits can build up in pressure tanks, water heaters, and pipelines, reducing the effective flow rate and efficiency of the water supply.

Iron was detected in the sample from your water well at 0.72 mg/l, which is above the CDPHE standard for drinking water.  Iron was not detected in the water sample from your well for the April 9 2004 sampling.

- Selenium (Se): The CDPHE selenium standard for human drinking water is 0.05 mg/l and the agricultural standard is 0.02 mg/l.  Excessive selenium (Se) (concentrations greater than 0.05 mg/l) can cause loss of hair and/or fingernails as well as adverse effects on the central nervous system.  Selenium (Se) occurs naturally in the ground water in many areas of Colorado at concentrations that exceed the drinking water standard.

Selenium was detected in the water sample from your well at a concentration of mg/l 0.093, which is above the human health standard for drinking water.  Selenium was detected in the water sample from your well for the April 9 2004 sampling at a concentration of 0.10 mg/l.

Several other metals, such as arsenic and calcium, were also detected in the water sample.  The following table presents a summary of these data.  Some of these metals do not have drinking water standards.

Table 1

| Metal | Gagne/Jacobsen Water Well | Drinking Water | Secondary Drinking Water Standard or Agricultural Standard |
| --- | --- | --- | --- |
| Arsenic (As) | 0.0037 | 0.05 | 0.1 |
| Barium (Ba) | 0.042 | 2.0 | No Standard |
| Calcium (Ca) | 13 | No Standard | No Standard |
| Magnesium (Mg) | 0.65 | No Standard | No Standard |
| Manganese (Mn) | 0.090 | No Standard | 0.2 |
| Potassium (K) | 0.90 | No Standard | No Standard |

All of the concentrations presented on Table 1 are in mg/l.

ORGANIC COMPOUNDS ASSOCIATED WITH PETROLEUM HYDROCARBONS

(please see enclosed Evergreen Analytical METHOD E 602/SW 8021B, BTEX DATA REPORT)

- Benzene: CDPHE's basic ground water standard for benzene is 5 micrograms per liter (µg/l).  Benzene was not detected in the sample from your water well in this sampling or the April 9, 2004 sample event.

- Toluene: CDPHE's basic ground water standard for toluene is 1,000 µg/l.  Toluene was not detected in the sample from your water well well in this sampling or the April 9, 2004 sample event.

- Ethylbenzene: CDPHE's basic ground water standard for ethylbenzene is 680 µg/l.  Ethylbenzene was not detected in the sample from your water well well in this sampling or the April 9, 2004 sample event.

- Total Xylenes (sum of m,p, and o-xylene): CDPHE's basic ground water standard for total xylenes is 10,000 µg/l.  Total xylenes were not detected in the sample from your water well well in this sampling or the April 9, 2004 sample event.

METHANE GAS CONCENTRATION

(Please see enclosed Evergreen Analytical Laboratory METHANE REPORT)

- Methane was detected in the sample from your water well at 0.0060 mg/l.  The sample result for the April 9, 2004 sample was 0.027 mg/l.

Methane gas alone is physiologically inert and non-toxic to humans.  Normal breath exhalation contains 1 to 99 ppm of methane (parts per million [ppm] is the same units as mg/l).  The presence of methane in drinking water does not present a known health hazard to humans or other animals via ingestion; however, methane in domestic water supplies can be associated with undesirable and potentially serious side effects.

Methane gas dissolved in water “exsolves” when exposed to the atmosphere and dissipates rapidly because it is lighter than air.  This is often responsible for the “fizzing” observed in water wells that may contain methane gas.  If the methane occurs at a high enough concentration and if it is allowed to accumulate in a confined space, such as a well pit, crawl space, closet, etc., an explosion hazard can be established.  In addition, if methane concentrations in well water are high, then pockets of free gas form within the water and cause the well pump to cavitate and no longer bring water to the surface.

Methane gas is common in water wells in Colorado.  It occurs naturally and the source of the methane is commonly from one or more of the sources listed below.

- Methane is commonly found as a gas in coal or black (organic rich) shale seams in the subsurface.

- Methane is a byproduct of the decay of organic matter.

- As the result of extensive testing for methane gas in water wells throughout Colorado, concentrations of methane gas below 1 mg/l are generally considered harmless, with concern for possible hazards from the methane increasing at concentration levels in well water at 7 mg/l and higher.  When methane levels are 13 mg/l or above, treatment is highly recommended.  Treatment generally includes some form of aeration to remove the methane from well water and ventilation to allow the methane to safely dissipate outdoors rather than accumulate within your home, well house or other confined space.

GAS COMPOSITIONAL AND ISOTOPIC ANALYTICAL RESULTS

(see attached Isotech Laboratories, Inc. report)

The gas sample from your water well contained very little methane and the laboratory was unable to analyze the sample for stable isotopes.  Compositionally the gas from your water well is composed primarily of nitrogen and oxygen with minor amounts of argon, carbon dioxide, and methane.

CONCLUSION

Because your water exceeded the CDPHE drinking water health advisory for  total dissolved solids (TDS), sodium (Na), fluoride (F), chloride (Cl), Nitrate/Nitrite as N, iron (Fe), selenium (Se), and because you or your livestock and/or pets drink your water, then you may wish to discuss the possible health effects of continued consumption with your physician and/or veterinarian.  Information regarding possible health effects of consumption of water from your well is also available from Ms. Mary Meisner, the Garfield County Public Health Nurse/Public Health Officer.  Ms. Mesiner has an office in Rifle, Colorado (902 Taughenbaugh Blvd, Suite 104; phone No. 970– 625–5200).

Comparisons of the water sample collected on April 9, 2004 and the sample collected on June 30, 2004 show very little variation in analytical results.  Minor variations in chemical concentrations are normal ground waters as the result of changes in precipitation which recharges the water table aquifer.  The increasing concentration of nitrogen in your water well may be due to a variety of causes, including the result of increase in land usage (such as agricultural, septic systems, livestock, etc.) in your area.  There are means of treatment to decrease the total nitrogen concentration in your water and can include anion exchange, reverse osmosis, and home distillation systems.  You may wish to contact a firm that specializes in home water purification and treatment to discuss possible treatment options for your water supply.

If you have any questions or would like to discuss these matters further, please contact me at the COGCC in Denver via e-mail (robert.chesson@state.co.us or by phone at 303-894-2100, extension 112.

Respectfully,

Robert H. Chesson, C.P.G., P.G.

Environmental Protection Specialist

Enclosures

cc:	Brian Macke– COGCC w/o enclosures

Debbie Baldwin – COGCC w/o enclosures

Morris Bell – COGCC w/o enclosures

Jamie Adkins – COGCC w/o enclosures