April 21, 2004

Mr. Jim Eubanks

6534 County Road 331

Silt, Colorado 81652

RE:	Water Quality Analytical Results for Your Water Well

Garfield County, Colorado

COGCC Divide Creek Gas Seep Investigation

Dear Mr. Eubanks:

As you are aware, on April 8, 2004, Debbie Baldwin of the Colorado Oil and Gas Conservation Commission (COGCC) sampled your water well for evidence of possible impact from a natural gas production well release near your residence.  This release is the likely cause of the gas seep recently developed in West Divide Creek near your residence.   The COGCC has received the results for some of the analyses conducted on your well water and this letter presents these results.  In addition, I have included as brief summary of the gas release investigation to date (April 21, 2004).   The water well samples from your well were submitted to Evergreen Analytical Laboratory (Evergreen) in Wheat Ridge, Colorado, for analysis of inorganic chemical constituents, organic compounds associated with petroleum hydrocarbons, methane gas.  At this time we have only received analytical data for BTEX (an acronym for the organic chemical compounds benzene, ethylbenzene, toluene, and total xylenes) and dissolved methane gas.  These two chemical analyses provide the best indication of natural gas impacts to water.  We anticipate receipt of the remaining analytical test in the very near future and will send you those results as soon as they are received.

BTEX:   (please see enclosed Evergreen Analytical METHOD E 602/SW 8021B, BTEX DATA REPORT)

- Benzene: Benzene was not detected in your water well.

- Toluene: Toluene was not detected in your water well.

- Ethylbenzene: Ethylbenzene was not detected in your water well.

- Total Xylenes (sum of m,p, and o-xylene): Total xylenes were not detected in your water well.

METHANE GAS CONCENTRATION

(Please see enclosed Evergreen Analytical Laboratory METHANE REPORT)

- Methane was detected in the sample from your water well at 0.058 mg/l.

Methane gas is common in water wells in Colorado.  It occurs naturally and the source of the methane is commonly from one or more of the sources listed below.

- Methane is commonly found as a gas in coal or black (organic rich) shale seams in the subsurface.

- Methane is a byproduct of the decay of organic matter.

EnCana previously sampled your water well on January 27, 2003 prior to their drilling activity in your area.  At that time dissolved methane was detected in your well at a concentration of 0.06 mg/l so this indicates that the methane concentration in your water well has not changed as the result of the gas release.

Isotopic Data Analyses

As part of the investigation of the gas seep in West Divide Creek, the COGCC collected several gas samples from the creek and from the surrounding EnCana production wells.  The purpose to this gas sampling was to identify the source of the gas seeping into the creek.   These gas samples were submitted to Isotech Laboratories (Isotech) in Champaign, Illinois for a more specific laboratory analysis for the stable isotopes of carbon and composition of the gas.   I have included a copy of the data plot that the COGCC uses to evaluate the isotopic data.  It is the COGCC staff’s opinion that the West Divide Creek seep gas is identical to the gas produced from the Williams Fork Formation in the wells in the area. The presence of Williams Fork Formation gas in the creek is likely due to cementing problems encountered by EnCana during drilling and completing one or more of their gas wells.  Those cementing problems have been rectified, however, gas seepage will continue in the creek for the near future, although we anticipate continued decreases in gas seep volume and overall seep activity. I have included the analytical results sheet for these gas samples along with a couple of location maps showing COGCC sample locations.

CONCLUSION

Based on the analytical laboratory results for the water well sample your water well has not been impacted by either natural gas drilling and production activities or the release in your area.   We will be sending you another summary letter after receiving the remaining analytical results.

If you have any questions or would like to discuss these matters further, please contact me or Debbie Baldwin at the COGCC in Denver via e-mail (robert.chesson@state.co.us); Debbie.baldwin@state.co.us) or by phone at 303-894-2100, extension 112 (Bob) or 111 (Debbie).

Respectfully,

Robert H. Chesson, C.P.G., P.G.

Environmental Protection Specialist

Enclosures

cc:	Rich Griebling – COGCC w/o enclosures

Debbie Baldwin – COGCC w/o enclosures

Jamie Adkins – COGCC w/o enclosures

Brian Macke – COGCC w/o enclosures