July 21, 2004

Mr. Jim Eubanks

6534 County Road 331

Silt, Colorado 81652

RE:	Water Quality Analytical Results for Your Stock Pond

Section 11 – Township 7 South – Range 92 West

Garfield County, Colorado

COGCC Complaint No. 200056844

Dear Mr. Eubanks:

On June 29, 2004, Debbie Baldwin and Bob Chesson of the Colorado Oil and Gas Conservation Commission (COGCC) collected one (1) water quality samples and one (1) gas samples from the stock pond located adjacent to your pole barn.  You had observed bubbles in your stock pond and asked the COGCC to investigate if the bubbling was related to the recent natural gas release from EnCana Oil & Gas’s Schwartz 2-15B well.  The COGCC staff believes that this release is the cause of the gas seep recently developed in West Divide Creek.   The COGCC has received the results for the analyses conducted on the water samples and methane gas composition and stable isotope sample and this letter presents these results.

The water samples from the EB Pond were submitted to Evergreen Analytical Laboratory (Evergreen) in Wheat Ridge, Colorado, for analysis of inorganic chemical constituents, organic compounds associated with petroleum hydrocarbons, and methane gas.  The gas composition and stable isotope samples were submitted to Isotech Laboratories (Isotech) in Champaign, Illinois.  Copies of the laboratory analytical reports are enclosed. I have also included a sample location map and a photograph of the pond from June 29, 2004.

The Water Quality Control Commission (WQCC) of the Colorado Department of Public Health and Environment (CDPHE) has established drinking water standards for the protection of human health. The analytical results from these water samples have been compared to applicable ground water and/or drinking water standards and are summarized below.  Please keep in mind that these water standards were established for public drinking water supplies and not for surface water ponds.

### COMPARISON OF INORGANIC ANALYTICAL RESULTS TO STANDARDS

(Please see enclosed Evergreen Analytical Laboratory Report)

- Total Dissolved Solids (TDS):  CDPHE has established a TDS standard for human drinking water of 500 milligrams per liter (mg/l).  Although CDPHE does not have an agricultural standard for TDS, other agencies recommend concentrations below 2,000 mg/l for irrigation, and below 5,000 mg/l for most livestock watering.  TDS concentrations are related to the presence of naturally occurring elements and chemical compounds such as chloride, sodium, potassium, calcium, magnesium, and sulfate.

##### TDS was detected in your stock pond at a concentration of 114 mg/l; which is within the CDPHE human drinking water standard, below the recommended maximum concentration for irrigation, and below the recommended maximum concentration for most livestock watering.

- Sodium (Na):  Although CDPHE does not have a standard for sodium, people on salt restricted diets should be aware of the Na concentration in the water they drink.  A concentration of drinking water with a concentration of sodium less than 20 mg/l is recommended by some for people on salt restricted diets or for people suffering from hypertension or heart disease.   Sodium occurs naturally in the ground water in many areas at concentrations that exceed the recommended level.

Sodium was detected in your stock pond at a concentration of 8.3 mg/l (EB Pond); which is within the recommended concentration for people on salt restricted diets.

##### Fluoride (F):  CDPHE has established a fluoride (F) standard for human drinking water is 4.0 mg/l.  Where fluoride concentrations are in the range of 0.7 mg/l to 1.2 mg/l, health benefits such as reduced dental decay have been observed.  Consumption of fluoride at concentrations of greater than 2.0 mg/l can result in mottling of teeth.  Consumption of fluoride at concentrations greater than 4.0 mg/l can increase the risk of skeletal fluorosis or other adverse health effects.

##### Fluoride was not detected in the sample from your stock pond.

##### Chloride (Cl):  The CDPHE chloride standard for drinking water is 250 mg/l.   Chloride concentrations in excess of 250 mg/l usually produce a noticeable taste in drinking water.

Chloride was not detected in the sample from your stock pond.

- Sulfate (SO4):  The CDPHE sulfate standard for drinking water is 250 mg/l.  Although CDPHE does not have an agricultural standard for sulfate, other agencies recommend a concentration below 1,500 mg/l for livestock watering.  Waters containing high concentrations of sulfate, typically caused by the leaching of natural deposits of magnesium sulfate (Epsom salts) or sodium sulfate (Glauber's salt), may be undesirable because of their laxative effects. Sulfate occurs naturally in the ground and surface water in many areas in Colorado at concentrations that exceed the drinking water standard.

Sulfate was detected in the sample from your stock pond at a concentration of 3.4 mg/l, which is within the CDPHE human drinking water standard and below the recommended maximum concentration for most livestock watering.

- Total Nitrate (NO3) + Nitrite (NO2) as Nitrogen (N):  The CDPHE total nitrate (NO3) + nitrite (NO2) as nitrogen (N) for standard for human drinking water is 10 mg/l.  Nitrate and nitrite are common contaminants in ground and surface water from agricultural sources, such as fertilizer and animal, including human, wastes.  They are known to cause infant cyanosis or “blue baby disease” in humans and, at concentrations greater than 100 mg/l as nitrogen (N), may be dangerous to livestock.  High concentrations of nitrate and nitrite in ground and surface water are known to occur in agricultural areas in Colorado.

Total nitrate/nitrite as N was detected in the in the sample from your stock pond at a concentration of 1.2 mg/l (EB Pond), which is within the CDPHE human drinking water standard.

- Iron (Fe):  The CDPHE standard for human drinking water for iron is 0.3 mg/l.  Small amounts of iron are common in ground water and surface waters.  Iron may produce a brownish-red color in laundered clothing, can leave reddish stains on fixtures, and impart a metallic taste to beverages and food made with it.  After a period of time iron deposits can build up in pressure tanks, water heaters, and pipelines, reducing the effective flow rate and efficiency of the water supply.

Iron was detected in the in the sample from your stock pond at a concentration of 0.43, which is slightly above the CDPHE human drinking water standard.

- Selenium (Se): The CDPHE selenium standard for human drinking water is 0.05 mg/l and the agricultural standard is 0.02 mg/l.  Excessive selenium (Se) (concentrations greater than 0.05 mg/l) can cause loss of hair and/or fingernails as well as adverse effects on the central nervous system.  Selenium (Se) occurs naturally in the ground water and surface water and, in many areas of Colorado, can occur at concentrations that exceed the drinking water standard.

Selenium was not detected in the sample from your stock pond.

Several other metals, such as barium and calcium, were also detected in the water sample.  The following table presents a summary of these data.  Some of these metals do not have drinking water standards.

Table 1

| Metal | Eubanks Stock Pond | Drinking Water | Secondary Drinking Water Standard or Agricultural Standard |
| --- | --- | --- | --- |
| Arsenic (As) | 0.0040 | 0.05 | 0.1 |
| Barium (Ba) | 0.095 | 2.0 | No Standard |
| Calcium (Ca) | 21 | No Standard | No Standard |
| Magnesium (Mg) | 4.6 | No Standard | No Standard |
| Manganese (Mn) | 0.050 | No Standard | 0.2 |
| Potassium (K) | 3.5 | No Standard | No Standard |

All of the concentrations presented on Table 1 are in mg/l.

ORGANIC COMPOUNDS ASSOCIATED WITH PETROLEUM HYDROCARBONS

(please see enclosed Evergreen Analytical METHOD E 602/SW 8021B, BTEX DATA REPORT)

- Benzene: CDPHE's basic ground water standard for benzene is 5 micrograms per liter (µg/l).  The CDPHE has a different benzene standard for surface water.   This benzene standard is 1.2 μg/l.

Benzene was not detected in the sample from your stock pond.

- Toluene: CDPHE's basic ground water standard for toluene is 1,000 µg/l.

Toluene was not detected in the sample from your stock pond.

- Ethylbenzene: CDPHE's basic ground water standard for ethylbenzene is 680 µg/l.

Ethylbenzene was not detected in the sample from your stock pond.

- Total Xylenes (sum of m,p, and o-xylene): CDPHE's basic ground water standard for total xylenes is 10,000 µg/l.

Total xylenes were not detected in the sample from your stock pond.

METHANE GAS CONCENTRATION

(Please see enclosed Evergreen Analytical Laboratory METHANE REPORT)

- Dissolved methane gas was detected in the sample from your stock pond at a concentration of 0.061 mg/l.

Methane gas alone is physiologically inert and non-toxic to humans.  Normal breath exhalation contains 1 to 99 ppm of methane (parts per million [ppm] is the same units as mg/l).  The presence of methane in drinking water does not present a known health hazard to humans or other animals via ingestion; however, methane in domestic water supplies can be associated with undesirable and potentially serious side effects.

Methane gas dissolved in water “exsolves” when exposed to the atmosphere and dissipates rapidly because it is lighter than air.  This is often responsible for the “fizzing” observed in water wells that may contain methane gas.  If the methane occurs at a high enough concentration and if it is allowed to accumulate in a confined space, such as a well pit, crawl space, closet, etc., an explosion hazard can be established.  In addition, if methane concentrations in well water are high, then pockets of free gas form within the water and cause the well pump to cavitate and no longer bring water to the surface.

Methane gas is common in water wells and surface water impoundments in Colorado.  It occurs naturally and the source of the methane is usually one of the following:

- Methane is commonly found as a gas in coal or black (organic rich) shale seams in the subsurface.

- Methane is a byproduct of the decay of organic matter.

GAS COMPOSITIONAL AND ISOTOPIC ANALYTICAL RESULTS

(see attached Isotech Laboratories, Inc. report)

A gas samples for determination of gas composition and stable isotopes of methane was collected from your stock pond on June 29, 2004.  The ratios of the stable isotopes of the carbon in the methane gas and detailed compositional analysis of the gases can be used to differentiate the origin and sources of methane.  The results of the isotopic and compositional analysis are summarized on the Isotech Laboratories report.

Isotopic Analysis of Methane:

The deuterium/hydrogen isotope ratio for the methane in the water sample from your stock pond is –363.0.

The carbon-13/carbon-12 isotope ratio for the methane in the water sample from your stock pond sample is -52.71.

The methane gas in your stock pond is biogenic methane, which is formed by bacteriological activity. Specifically gas researchers have identified as the type of biogenic methane found in your pond as Near Surface Microbial Gas as the result of biological fermentation, and is very common in surface ponds in temperate climates.   I have included a cross-plot of the stable methane isotopes for the gas samples from your pond.  You will notice the area near the top right corner as defined a “Thermogenic Gas”.  This is the area of the cross-plot that the natural gas produced by the gas wells in the Mamm Creek Gas Field (and the gas present in the West Divide Creek Gas seep) plot.  Thermogenic means methane gas produced by geologic processes.

The gas compositional analysis indicates that the gas sample collected from your pond is made up of a mixture of air (nitrogen, oxygen, etc.) and methane. No other amounts of “heavier” hydrocarbon gases (such as propane or butane) are present. This is also very common in gas produced by biological activity.  If the methane gas in the EB Pond was due to a release from a gas production well, not only would the gas cross-plot as thermogenic, the gas composition would contain “heavier” hydrocarbon gases (as is seen in the gas from the West Divide Creek Gas seep).

CONCLUSION

Methane gas stable isotope and gas composition results for the sample from your stock pond shows that the bubbling observed is due to natural biogenic methane gas produced by bacteria.  Additionally, overall water quality of your pond show chemical conditions of your pond to be within conditions that would be expected for a surface water stock pond or impoundment.

If you have any questions or would like to discuss these matters further, please contact me at the COGCC in Denver via e-mail (robert.chesson@state.co.us), or by phone at 303-894-2100, extension 112.

Respectfully,

Robert H. Chesson, C.P.G., P.G.

Environmental Protection Specialist

Enclosures

cc:	Debbie Baldwin – COGCC w/o enclosures

Jamie Adkins – COGCC w/o enclosures

Brian Macke – COGCC w/o enclosures

Morris Bell – COGCC w/o enclosures