July 19, 2004

Ms. Lisa Bracken

Mr. Robert Eicher

P.O. Box 30

Silt, Colorado 81652

RE:	Water Quality Analytical Results for EB Pond Samples

Section 12 – Township 7 South – Range 92 West

Garfield County, Colorado

West Divide Creek Gas Seep Investigation

COGCC Remediation No. 1815

Dear Ms. Bracken and Mr. Eicher:

As you are aware, on June 29, 2004, Debbie Baldwin and Bob Chesson of the Colorado Oil and Gas Conservation Commission (COGCC) collected two (2) water quality samples and two (2) gas samples from the EB Pond (formerly the “Beaver Pond”) located within the main channel of West Divide Creek on your property.  The EB Pond was sampled for evidence of possible impact from the natural gas release from EnCana Oil & Gas’s Schwartz 2-15B well.  The COGCC staff believes that this release is the cause of the gas seep recently developed in West Divide Creek.   The COGCC has received the results for the analyses conducted on these water samples and methane gas composition and stable isotope samples and this letter presents these results.

The water samples from the EB Pond were submitted to Evergreen Analytical Laboratory (Evergreen) in Wheat Ridge, Colorado, for analysis of inorganic chemical constituents, organic compounds associated with petroleum hydrocarbons, and methane gas.  The gas composition and stable isotope samples were submitted to Isotech Laboratories (Isotech) in Champaign, Illinois.  Copies of the laboratory analytical reports are enclosed.  I am still waiting on the bacteriological analysis result for two (2) samples of the biofilm, one (1) collected from Mr. Thompson’s culvert and one (1) collected from the eastern bank of West Divide Creek adjacent to the main gas seep.  I will forward these sample results as soon as they are available.  A map showing the sample locations is included with this letter.

The Water Quality Control Commission (WQCC) of the Colorado Department of Public Health and Environment (CDPHE) has established drinking water standards for the protection of human health. The analytical results from these water samples have been compared to applicable ground water and/or drinking water standards and are summarized below.  Please keep in mind that these water standards were established for public drinking water supplies and not for surface water ponds.   The EB Pond has been sampled for water quality five times (5); On April 8, 2004 (by both the COGCC and EnCana), on May 19, 2004 (COGCC), and the most recent samplings (2)  on June 29, 2004.  I have provided a comparison of the data from the April 8, 2004 and June 29, 2004 samples in the following discussion.

### COMPARISON OF INORGANIC ANALYTICAL RESULTS TO STANDARDS

(Please see enclosed Evergreen Analytical Laboratory Report)

- Total Dissolved Solids (TDS):  CDPHE has established a TDS standard for human drinking water of 500 milligrams per liter (mg/l).  Although CDPHE does not have an agricultural standard for TDS, other agencies recommend concentrations below 2,000 mg/l for irrigation, and below 5,000 mg/l for most livestock watering.  TDS concentrations are related to the presence of naturally occurring elements and chemical compounds such as chloride, sodium, potassium, calcium, magnesium, and sulfate.

##### TDS was detected in the EB Pond samples at a concentration of 386 mg/l (EB Pond) and 386 mg/l (EB Pond 2), both collected on June 29, 2004, and  at a concentration of 279 mg/l for the sample collected on April 8, 2004.  All sample results are within the CDPHE human drinking water standard, below the recommended maximum concentration for irrigation, and below the recommended maximum concentration for most livestock watering.

- Sodium (Na):  Although CDPHE does not have a standard for sodium, people on salt restricted diets should be aware of the Na concentration in the water they drink.  A concentration of drinking water with a concentration of sodium less than 20 mg/l is recommended by some for people on salt restricted diets or for people suffering from hypertension or heart disease.   Sodium occurs naturally in the ground water in many areas at concentrations that exceed the recommended level.

Sodium was detected in the EB Pond samples at a concentration of 60 mg/l (EB Pond) and 56 mg/l (EB Pond 2), both collected on June 29, 2004, and at a concentration of 36 “B” mg/l for the sample collected on April 8, 2004. All samples collected have sodium concentration greater than the recommended level for people on salt restricted diets. The B notation is a laboratory “flag” which is used to indicate the presence of a laboratory-induced contaminant in the April 8, 2004 sample (known as blank [“B”] contamination).  In this case the laboratory noticed the presence of sodium in the analytical method control sample (called a method blank).  Normally the method blank does not have any compounds.  The actual concentration of sodium in the April 8, 2004 sample would be 36 mg/l less the blank detection (0.3182 mg/l) or 35.6818 mg/l.

##### Fluoride (F):  CDPHE has established a fluoride (F) standard for human drinking water is 4.0 mg/l.  Where fluoride concentrations are in the range of 0.7 mg/l to 1.2 mg/l, health benefits such as reduced dental decay have been observed.  Consumption of fluoride at concentrations of greater than 2.0 mg/l can result in mottling of teeth.  Consumption of fluoride at concentrations greater than 4.0 mg/l can increase the risk of skeletal fluorosis or other adverse health effects. 
Fluoride was detected in the EB Pond samples at a concentration of 0.59 mg/l (EB Pond) and 0.59 mg/l (EB Pond 2), both collected on June 29, 2004, and at a concentration of 0.44 mg/l for the sample collected on April 8, 2004.  All sample results are within the CDPHE human drinking water standard.

##### Chloride (Cl):  The CDPHE chloride standard for drinking water is 250 mg/l.   Chloride concentrations in excess of 250 mg/l usually produce a noticeable taste in drinking water.

Chloride was detected in the EB Pond samples at a concentration of 8.28 mg/l (EB Pond) and 8.37 mg/l (EB Pond 2), both collected on June 29, 2004, and at a concentration of 4.58 mg/l for the sample collected on April 8, 2004.  All sample results are within the CDPHE human drinking water standard.

- Sulfate (SO4):  The CDPHE sulfate standard for drinking water is 250 mg/l.  Although CDPHE does not have an agricultural standard for sulfate, other agencies recommend a concentration below 1,500 mg/l for livestock watering.  Waters containing high concentrations of sulfate, typically caused by the leaching of natural deposits of magnesium sulfate (Epsom salts) or sodium sulfate (Glauber's salt), may be undesirable because of their laxative effects. Sulfate occurs naturally in the ground and surface water in many areas in Colorado at concentrations that exceed the drinking water standard.

Sulfate was detected in the EB Pond samples at a concentration of 43.6 mg/l (EB Pond) and 43.1 mg/l (EB Pond 2), both collected on June 29, 2004, and at a concentration of 21.2 mg/l for the sample collected on April 8, 2004.  All sample results are within the CDPHE human drinking water standard and below the recommended maximum concentration for most livestock watering.

- Total Nitrate (NO3) + Nitrite (NO2) as Nitrogen (N):  The CDPHE total nitrate (NO3) + nitrite (NO2) as nitrogen (N) for standard for human drinking water is 10 mg/l.  Nitrate and nitrite are common contaminants in ground and surface water from agricultural sources, such as fertilizer and animal, including human, wastes.  They are known to cause infant cyanosis or “blue baby disease” in humans and, at concentrations greater than 100 mg/l as nitrogen (N), may be dangerous to livestock.  High concentrations of nitrate and nitrite in ground and surface water are known to occur in agricultural areas in Colorado.

Total nitrate/nitrite as N was detected in the in the EB Pond samples at a concentration of 0.30 mg/l (EB Pond) and 3.45 mg/l (EB Pond 2), both collected on June 29, 2004.  Total nitrate/nitrite, as N was not detected in the water sample collected on April 8, 2004.  All sample results are within the CDPHE human drinking water standard.

- Iron (Fe):  The CDPHE standard for human drinking water for iron is 0.3 mg/l.  Small amounts of iron are common in ground water and surface waters.  Iron may produce a brownish-red color in laundered clothing, can leave reddish stains on fixtures, and impart a metallic taste to beverages and food made with it.  After a period of time iron deposits can build up in pressure tanks, water heaters, and pipelines, reducing the effective flow rate and efficiency of the water supply.

Iron was detected in the in the EB Pond samples at a concentration of 0.35 mg/l (EB Pond) and at 1.1 mg/l (EB Pond 2), both collected on June 29, 2004.  Iron was not detected in the water sample collected on April 8, 2004.  All sample results are within the CDPHE human drinking water standard.

- Selenium (Se): The CDPHE selenium standard for human drinking water is 0.05 mg/l and the agricultural standard is 0.02 mg/l.  Excessive selenium (Se) (concentrations greater than 0.05 mg/l) can cause loss of hair and/or fingernails as well as adverse effects on the central nervous system.  Selenium (Se) occurs naturally in the ground water and surface water and, in many areas of Colorado, can occur at concentrations that exceed the drinking water standard.

Selenium was detected in the in the EB Pond samples at a concentration of 0.0061 mg/l (EB Pond) and at 0.0064 mg/l (EB Pond 2), both collected on June 29, 2004.   Selenium was not detected in the water sample collected on April 8, 2004.  All sample results are within the CDPHE human drinking water standard.

Several other metals, such as barium and calcium were also detected in the water samples.  The following table presents a summary of these data.  Some of these metals do not have drinking water standards.

Table 1

| Metal | EB Pond | EB Pond 2 | Drinking Water | Secondary Drinking Water Standard or Agricultural Standard |
| --- | --- | --- | --- | --- |
| Barium (Ba) | 0.13 | 0.14 | 2.0 | No Standard |
| Calcium (Ca) | 55 | 56 | No Standard | No Standard |
| Magnesium (Mg) | 23 | 23 | No Standard | No Standard |
| Manganese (Mn) | 0.043 | 0.090 | No Standard | 0.2 |
| Potassium (K) | 3.2 | 3.4 | No Standard | No Standard |

All of the concentrations presented on Table 1 are in mg/l.

ORGANIC COMPOUNDS ASSOCIATED WITH PETROLEUM HYDROCARBONS

(please see enclosed Evergreen Analytical METHOD E 602/SW 8021B, BTEX DATA REPORT)

- Benzene: CDPHE's basic ground water standard for benzene is 5 micrograms per liter (µg/l). Because West Divide Creek is classified by WQCC as a Water Supply Segment for the Lower Colorado River Basin, the CDPHE has a different benzene standard for surface water.   This benzene standard is 1.2 μg/l.

Benzene was not detected in the in the either of the EB Pond samples (EB Pond) and  (EB Pond 2), both collected on June 29, 2004.   Benzene was also not detected in the water sample collected on April 8, 2004.

- Toluene: CDPHE's basic ground water standard for toluene is 1,000 µg/l.

Toluene was not detected in the in the either of the EB Pond samples (EB Pond) and  (EB Pond 2), both collected on June 29, 2004.   Toluene was also not detected in the water sample collected on April 8, 2004.

- Ethylbenzene: CDPHE's basic ground water standard for ethylbenzene is 680 µg/l.

Ethylbenzene was not detected in the in the either of the EB Pond samples (EB Pond) and  (EB Pond 2), both collected on June 29, 2004.   Ethylbenzene was also not detected in the water sample collected on April 8, 2004.

- Total Xylenes (sum of m,p, and o-xylene): CDPHE's basic ground water standard for total xylenes is 10,000 µg/l.

Total xylenes were not detected in the in the either of the EB Pond samples (EB Pond) and  (EB Pond 2), both collected on June 29, 2004.   Total xylenes were also not detected in the water sample collected on April 8, 2004.

METHANE GAS CONCENTRATION

(Please see enclosed Evergreen Analytical Laboratory METHANE REPORT)

Table 2 presents a summary of the dissolved methane gas results for both COGCC and EnCana Samples from the EB Pond.

TABLE 2

| SAMPLE DATE | SAMPLE ID | DISSOLVED METHANE CONCENTRATION (mg/l) |
| --- | --- | --- |
| April 8, 2004 | COGCC ENCANA | 0.025 0.015 |
| June 29, 2004 | EB Pond EB Pond 2 | 0.0081 0.012 |

Methane gas alone is physiologically inert and non-toxic to humans.  Normal breath exhalation contains 1 to 99 ppm of methane (parts per million [ppm] is the same units as mg/l).  The presence of methane in drinking water does not present a known health hazard to humans or other animals via ingestion; however, methane in domestic water supplies can be associated with undesirable and potentially serious side effects.

Methane gas dissolved in water “exsolves” when exposed to the atmosphere and dissipates rapidly because it is lighter than air.  This is often responsible for the “fizzing” observed in water wells that may contain methane gas.  If the methane occurs at a high enough concentration and if it is allowed to accumulate in a confined space, such as a well pit, crawl space, closet, etc., an explosion hazard can be established.  In addition, if methane concentrations in well water are high, then pockets of free gas form within the water and cause the well pump to cavitate and no longer bring water to the surface.

Methane gas is common in water wells and surface water impoundments in Colorado.  It occurs naturally and the source of the methane is usually one of the following:

- Methane is commonly found as a gas in coal or black (organic rich) shale seams in the subsurface.

- Methane is a byproduct of the decay of organic matter.

GAS COMPOSITIONAL AND ISOTOPIC ANALYTICAL RESULTS

(see attached Isotech Laboratories, Inc. report)

Gas samples for determination of gas composition and stable isotopes of methane were collected from the EB Pond on June 29, 2004.  There was no gas isotope sample collected from the EB Pond on April 8, 2004 due to lack of obvious gas bubbling.  The COGCC attempted to collect a methane gas sample from the EB Pond on May 19, 2004; however, subsequent analysis of the gas sample collected showed insufficient methane gas for isotope or compositional analyses.

The ratios of the stable isotopes and detailed compositional analysis of the gases can be used to differentiate the origin and sources of methane.  The results of the isotopic and compositional analysis are summarized on the Isotech Laboratories report.

EB Pond

Isotopic Analysis of Methane:

The deuterium/hydrogen isotope ratio for the methane in the water sample from the EB Pond sample is –394.3.

The carbon-13/carbon-12 isotope ratio for the methane in the water sample from the EB Pond sample is -57.00.

EB Pond 2

Isotopic Analysis of Methane:

The deuterium/hydrogen isotope ratio for the methane in the water sample from the EB Pond 2 sample is –398.1.

The carbon-13/carbon-12 isotope ratio for the methane in the water sample from the EB Pond sample is -57.55.

The gas in the EB Pond is biogenic methane, which is formed by bacteriological activity. Specifically gas researchers have identified as the type of biogenic methane found in the EB Pond as Near Surface Microbial Gas as the result of biological fermentation, and is very common in surface ponds in temperate climates.   I have included a cross-plot of the stable methane isotopes for the gas samples from the EB Pond.  You will notice the area near the top right corner as defined a “Thermogenic Gas”.  This is the area of the cross-plot that the natural gas produced by the gas wells in the Mamm Creek Gas Field (and the gas present in the West Divide Creek Gas seep) plot.  Thermogenic means methane gas produced by geologic processes.

The gas compositional analysis indicates that the gas samples collected from the EB Pond are made up of a mixture of air (nitrogen, oxygen, etc.) and methane. No other amounts of “heavier” hydrocarbon gases (such as propane or butane) are present. This is also very common in gas produced by biological activity.  If the methane gas in the EB Pond was due to a release from a gas production well, not only would the gas cross-plot as thermogenic, the gas composition would contain  “heavier” hydrocarbon gases (as is seen in the gas from the West Divide Creek Gas seep).

CONCLUSION

With the exception of sodium (Na) no other organic or inorganic compounds detected in the either the April 8, 2004 or the June 29, 2004 sampling of the EB Pond exceed any of the CDPHE drinking water health advisories or standards.  Although the concentrations of sodium detected are relatively low, because you or your livestock and pets may drink water form the creek, you may wish to discuss the possible health effects of continued consumption with your physician and/or veterinarian.

Comparisons of the water sample collected on April 8, 2004 and the two (2) water samples collected on June 29, 2004 show very little variation in analytical results.  Minor variations in chemical concentrations are normal in surface water bodies as the results of seasonal changes in water volumes and flows.

As discussed earlier, the methane gas isotopic and compositional samples collected (June 29, 2004) demonstrate that the methane gas in the EB Pond is the result of natural biological activity and is not related to the gas release from the Schwartz 2-15B well.

In response to your concerns regarding bubbling in the EB Pond, COGCC staff has visited your pond on three different times (April 8, 2004, May 19, 2004, and June 29, 2004).  On none of these visits have staff observed anything unusual.  The small, intermittent bubbling that has been observed (and sampled on June 29, 2004) is similar in appearance and frequency to bubbling we have commonly observed in our official capacity during other complaint responses.  It is in our opinion, the result of natural biological processes common to surface water bodies throughout temperate parts of the United States.  The result of the isotopic and water quality analysis of the water and gas in the EB Pond confirms our observations and opinion.

If you have any questions or would like to discuss these matters further, please contact me in Denver via e-mail (robert.chesson@state.co.us); or by phone at 303-894-2100, extension 112.

Respectfully,

Robert H. Chesson, C.P.G., P.G.

Environmental Protection Specialist

Enclosures

cc:	Brian Macke – COGCC w/o enclosures

Debbie Baldwin – COGCC w/o enclosures

Jamie Adkins – COGCC w/o enclosures

Morris Bell – COGCC w/o enclosures