April 30, 2004

Mr. Mike Smith

795 County Road 326

Silt, Colorado 81652

RE:	Water Quality Analytical Results for Your Water Well

Section 35 – Township 6 South – Range 92 West

Garfield County, Colorado

West Divide Creek Gas Seep Investigation

COGCC Investigation No. 1802

Dear Mr. Smith:

As discussed in my April 26, 2004 letter, the Colorado Oil and Gas Conservation Commission (COGCC) had not yet received the final analytical results from Evergreen Analytical Laboratory (Evergreen) for the April 9, 2004 sampling of your water well.  We received the final analytical results on April 29, 2004 and this letter presents the results for Total Dissolved Solids (TDS) and a revision of the result for sodium (Na) which was previously reported.  The preliminary report for sodium concentration in your well identified laboratory induced blank contamination; however, the magnitude of the blank contamination was not included in the preliminary report.  This letter corrects the sodium concentration result for the blank contamination.  As with my previous letter, the analytical results from the water samples from your well have been compared to applicable ground water and/or drinking water standards and are summarized below.  These water standards were established for public drinking water supplies and often people use and consume ground water from private wells that can exceed these standards.

- Total Dissolved Solids (TDS):  CDPHE has established a TDS standard for human drinking water of 500 milligrams per liter (mg/l).  Although CDPHE does not have an agricultural standard for TDS, other agencies recommend concentrations below 2,000 mg/l for irrigation, and below 5,000 mg/l for most livestock watering.  TDS concentrations are related to the presence of naturally occurring elements and chemical compounds such as chloride, sodium, potassium, calcium, magnesium, and sulfate.

##### TDS was detected in the water sample from your well at a concentration of 1,480

mg/l.  This is above the CDPHE human drinking water standard, below the recommended maximum concentration for irrigation, and below the recommended maximum concentration for most livestock watering.

- Sodium (Na):  Although CDPHE does not have a standard for sodium, people on salt restricted diets should be aware of the Na concentration in the water they drink.  A concentration of drinking water with a concentration of sodium less than 20 mg/l is recommended by some for people on salt restricted diets or for people suffering from hypertension or heart disease.   Sodium occurs naturally in the ground water in many areas at concentrations that exceed the recommended level.

Sodium was detected in the water sample from your well at a concentration of 490 “B”  mg/l, which is greater than the recommended level for people on salt restricted diets. The B notation is a laboratory “flag” which is used to indicate the presence of a laboratory-induced contaminant in your sample (known as blank [“B”] contamination).  In this case the laboratory noticed the presence of sodium in the analytical method control sample (called a method blank).    The actual concentration of sodium in your water well sample would be 490 mg/l less the blank detection (0.3182 mg/l) or 489.682 mg/l

If you have any questions or would like to discuss these matters further, please contact me or Debbie Baldwin at the COGCC in Denver via e-mail (robert.chesson@state.co.us); Debbie.baldwin@state.co.us) or by phone at 303-894-2100, extension 112 (Bob) or 111 (Debbie).

Respectfully,

Robert H. Chesson, C.P.G., P.G.

Environmental Protection Specialist

Enclosure

cc:	Rich Griebling – COGCC w/o enclosure

Debbie Baldwin – COGCC w/o enclosure

Jamie Adkins – COGCC w/o enclosure

Brian Macke – COGCC w/o enclosure