Garfield County Request For Proposal - Data Download

All sample locations and analytical data in the COGCC analytical database for townships 6S and 7S and for ranges 92W and 93W were extracted to ASCII files on 4/14/05.  Four files are available on the website:

- OGwells6_7S92_93W.csv – (Oil & Gas wells) For posting identifying info and coordinates for wells with API numbers.

- OGAnalyses6_7S92_93W.csv – For downloading analytical results for samples collected from Oil and Gas wells.

- WWells6_7S92_93W.csv – (Water wells) For posting identifying info and coordinates for water wells or other sample locations without API numbers, including water wells, surface water, springs, ponds, etc..

- WWAnalyses6_7S92_93W.csv – For downloading analytical results for samples collected from water wells or other sample locations without API numbers, including water wells, surface water, springs, ponds, etc.

Data is downloadable in comma delimited text files (*.CSV).  Text fields are delimited by double quotes (“) and commas.  The best way to view and manipulate the data is to import the files into a database program and link the tables on the facility_id fields.

Data structures

OGwells6_7S92_93W.csv – List of all Oil and Gas wells in the selected area (including wells not sampled).  Database fields are:

FieldName	Type	Size	Description

api_county_code	Text	3	3 digit code designating county (OG wells only)

api_seq_num	Text	5	In combination with api_county_code, a unique well identifier assigned by COGCC to oil and gas wells in Colorado (OG wells only)

facility_id	Integer		Unique identifier for sample point, assigned by COGCC

WellName	Text	50	Concatenation of COGCC facility_name and facility_num fields, assigned to oil and gas wells

Utm_x, utm_y	Integer		Well location coordinates in meters, Universal Transverse Mercator, NAD 27, Zone 13N.

Sec	Text	2	Section number

Twp	Text	6	Township number and direction (eg “6S”)

Range	Text	7	Range number and direction (eg “92W”)

Meridian	Text	1	Survey Meridian

Facility_status	Text	2	Current facility status

OGwells6_7S92_93W.csv (continued)

| Status | Description |
| --- | --- |
| AC | Active |
| AL | Abandoned Location |
| CM | Comingled |
| DA | Dry and Abandoned |
| DG | Drilling |
| PA | Plugged and Abandoned |
| PR | Producing |
| RC | Recompleted |
| SI | Shut in |
| TA | Temporarily Abandoned |
| UN | Unitized (several leases combined into one unit) |
| VP | Verbal intent to plug |
| WO | Waiting on completion |
| XX | Approved permit to drill |

Ground_elev	Number		Ground surface elevation (ft AMSL)

Operator_num	Number		Identifier assigned by COGCC

Name	Text	50	Operator name

WWells6_7S92_93W.csv – List of sampled water wells and other water sources (eg springs) in the selected area.  Database fields are:

FieldName	Type	Size	Description

facility_id	Integer		Unique identifier for sample point, assigned by COGCC

Ownername	Text	30	Name of well owner or other person or entity associated with the sample location

WellName	Text	50	Non-unique identifier for sample locations other than oil and gas wells (water wells, springs, etc.)

WWells6_7S92_93W.csv (continued)

Typewell	Text	3	Sample location type classification.  Where typewell is blank, the sample location is probably a water well.

| TYPEWELL | TYPENAME |
| --- | --- |
| CIS | CISTERN |
| DOM | DOMESTIC (HOME) WELL |
| MW | Mon/Obs well |
| PND | Gas from Pond |
| PND | Pond |
| S/G | Soil gas mon probe |
| SPR | Spring or Seep |
| WTR | Water Supply Well |

Utm_x, utm_y	Integer		Well location coordinates in meters, Universal Transverse Mercator, NAD 27, Zone 13N.

Twp	Text	6	Township number and direction (eg “6S”)

Range	Text	7	Range number and direction (eg “92W”)

Sec	Text	2	Section number

QtrQtr	Text	6	Quarter Quarter of section

Meridian	Text	1	Survey Meridian

County_code	Text	3	COGCC assigned code for county.  All locations are in Garfield County (county_code 045)

WellDepth	Number		Completed well depth, ft BGS

Ground_elev	Number		Ground surface elevation, ft AMSL

OGAnalyses6_7S92_93W.csv,

and WWAnalyses6_7S92_93W.csv – Analytical results for samples collected from Oil and Gas wells, or from non-Oil and Gas wells (water wells and other sample locations) respectively  Sample location identification is obtained by linking on facility_id to the OGwells6_7S92_93W.csv or WWells6_7S92_93W.csv tables.  Database fields are:

FieldName	Type	Size	Description

facility_id	Integer		Unique identifier for sample point, assigned by COGCC

Matrix	Text	1	Sample matrix: g-gas, w-water;

DATE	Date/Time		Format m/d/yyyy h:mm:ss

DUP	Text	3	Flag to distinguish multiple samples collected in same day (not necessarily duplicate samples).  Default value is “0”

ANALYTE	Text	50	Name of parameter analyzed

OGAnalyses6_7S92_93W.csv,

and WWAnalyses6_7S92_93W.csv

RESULT	Numeric		Detected concentration of analyte

CODE	Text	5	Qualifier for Analysis (list follows):

DETLIM	Numeric		Detection Limit (minimum detectable concentration in the sample)

UNITS	Text	15	Units of Concentration (applies to RESULT and DETLIM fields)

FmCode	Text	4	Code for formation sampled.

| FMCODE | description |
| --- | --- |
| ALVM | ALLUVIUM |
| MVRD | MESAVERDE |
| RLNS | ROLLINS |
| UNK | UNKNOWN |
| WMFK | WILLIAMS FORK |
| WSTC | WASATCH |

TypeSamp	Text	4	Type of well or type of sample classification.

| TypeSamp codes for OG wells |  |
| --- | --- |
| TYPESAMP | TYPENAME |
| BDH | Bradenhead gas |
| DST | DRILLSTEM |
| INT | Intermediate casing |
| PRD | Produced gas |
| PRD | Produced water |
| S/G | Soil gas mon probe |
| SEP | Separator/Water Tank |

| TypeSamp codes for WW samples |  |
| --- | --- |
| TYPESAMP | TYPENAME |
| CIS | CISTERN |
| DOM | DOMESTIC (HOME) WELL |
| MW | Mon/Obs well |
| PND | Gas from Pond |
| PND | Pond |
| RIV | River/Stream |
| S/G | Soil gas mon probe |
| SPR | Spring or Seep |
| WTR | Gas from Wtr Well |
| WTR | Water Supply |

SOURCE	Text	100	Source of information as compiled in COGCC database